AM2/CRS-A/D(e) SCRIPT MANUAL

for Analog/CAS VoP 1-Way Delay Origination Script

( 9327079D )

This document describes a script developed for the AM2/CRS-A/D(e) Bulk Call Generator. It consists of five parts.

| Part 1: A title and description of the script | ---Page 1 |
| --- | --- |
| Part 2: Call Flow | ---Page 2 |
| Part 3: Description of Script Parameters | ---Page 2,3 |
| Part 4: Explanation of Report Codes | ---Page 3 |
| Part 5: Report codes used by the One-way Delay test script | ---Page 4 |

| Script Number | Title Description | AM2-A/D(e) Selection |
| --- | --- | --- |
| #### 9327079D | Analog/CAS VoP 1-Way Delay Originate | UNPR O 1-Way |

WARNING:  This script is developed to be used on Analog, T1/E1 CAS units equipped with 25-0419 VoP Option.

REQUIREMENTS:

- Host Software Version 4.77a or higher.

- 25-0419-A/D(e) Option.

CALL FLOW

# The program calls the terminating line and checks for answer supervision.  After the voice path is established the program waits  for a test tone pulse from the terminating side.  When the pulse is received, the one-way propagation delay is measured and stored. A short delay is performed.(interpulse delay) Then the program sends the pulse, the terminating side measures and stores the result. (Please refer to the VoP Application User`s Guide for more information)  The calling line will go onhook after the testing is complete.

# SCRIPT PARAMETERS

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The analog line number of the terminating line. 1-160 A time slot in one of the four T1/E1 trunks in which voice will be sent once the call is connected. The acceptable range is A01 to D24 for a T1 unit and A01 to D31 on an E1 unit. A01 - D24, A01 - D31 |
| dial_type |  | Dial type. Types available: 1=Pulse , 2=DTMF, 3=MFR1, 4=MFR2. Normally keep this value at 2 for Analog/CAS Signaling. |
| digits_1 digits_2 digits_3 |  | The phone number that the channel will dial to ring the called line. digits_2 is used when more than the 15 digits of digits_1 are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. digits_3 is used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| answer_timer | 5.0 | Timeout value for answer supervision. |
| cut_thru | 2.0 | Time in seconds from the time the originate side establishes voice path and starts waiting for the test tone pulse coming from the termination line. |
| dial_delay | 0 | Number of seconds delay from the time dial tone is detected until start of dialing. |
| inter_pulse | 4 | The inter_pulse parameter selects the delay between the receive 1-way and the send 1-way delay tests on the unpaired originate script only. The inter_pulse parameter, along with the receive pulse timeout (rpulse_timer), on the terminate script can be modified together to increase the call holding time. The inter_pulse parameter should not be set lower than 4 seconds, since this will result in periodic test failures due to timing mismatches. |
| intercall | 4 | Time, in seconds, between the end of one call and the beginning of the next call. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_hi | 0 | Same as offset_lo, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_lo | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| rpulse_timer | 4 | The rpulse_timer parameter selects the timeout for the receive 1-way tone pulse. |
| spulse_tone | 14 | The spulse_tone parameter selects the tone that is sent during the pulse test. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to initate a call until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t initiate at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, the timed_start will only delay before the initial off hook, from issuance of a “start” command. |
| bin_1_max bin_2_max bin_3_max bin_4_max |  | The bin_max parameters are used to define the statistical bins which tally the dropout test results 1-Way Delay Tests. Each test counts the number of dropouts/lost packets detected during the dropout test stage. The number of dropout counts are calculated into the dropout average drops per call (CODE 2120). In addition, one of the statistical bins is incremented for each completed test. A count is scored in the bin in which the dropout count for the call falls. The bin dropout ranges are defined as follows: DROP_BIN_1 0 to bin1_max 0 DROP_BIN_2 (bin1_max + 5ms) to bin_2_max 01 - 50 DROP_BIN_3 (bin2_max + 5ms) to bin_3_max 51 - 100 DROP_BIN_4 (bin3_max + 5ms) to bin_4_max 101 - 250 DROP_BIN_5 (bin4_max + 5ms) and up 251 + |

REPORT CODES

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| R_ATTEMPT (CODE 1) | Origination Attempts | Counts each occurrence of an OFFHOOK occurrence regardless of the success of the call. |
| R_COMPLETE (CODE 2) | Origination Completes | Counts the number of times the programmed sequence was successful in running through its complete test. |
| R_STARTDELAY (CODE 3) | Average Start Delay | Average time it takes to receive dial tone. |
| R_SLOWSTART (CODE 4) | Slow Start Signal x | The expected signal to start dialing was not detected by the protocol within the delay timeout set by the 'st_sig_dly' parameter. |
| R_NOSTART (CODE 5) | No Start Signal x | The expected signal to start dialing was not detected by the protocol within the fail timeout set by the 'st_sig_fail' parameter. |
| R_PDTIME (CODE 7) | Avg PD Delay | Average Post Dial Delay. |
| R_NOCONFIRM (CODE 8) | No Confirm x | Number of times One Way Pulse test times out. |

Note: See your instruction manual or the Script Writing Guide for an explanation of other report codes which may occur.

REPORT CODES USED BY THE ONE-WAY DELAY TEST

2100 AVER_DELAY: Average 1-Way Delay. Reports the average one-way delay in milliseconds.

2101 MIN_DELAY: Minimum 1-Way Delay. Reports the minimum one-way delay in milliseconds.

2102 MAX_DELAY: Maximum 1-Way Delay. Reports the maximum one-way delay in milliseconds.

2103 DELAY_BIN_1: 1-Way Delay Bin 1 Counts. 100 - counts the number of calls where the one-way delay is less than 100 milliseconds.

2104 DELAY_BIN_2: 1-Way Delay Bin 2 Counts. 200 - counts the number of calls where the one-way delay is between 101 and 200 milliseconds.

2105 DELAY_BIN_3: 1-Way Delay Bin 3 Counts. 300 - counts the number of calls where the one-way delay is between 201 and 300 milliseconds.

2106 DELAY_BIN_4: 1-Way Delay Bin 4 Counts. 400 - counts the number of calls where the one-way delay is between 301 and 400 milliseconds.

2107 DELAY_BIN_5: 1-Way Delay Bin 5 Counts. Counts the number of calls where the one-way delay is greater than 401 milliseconds.