CRS & AM2 SCRIPT MANUAL

For Unpaired Terminate Golden Voice Basic (Analog and CAS) Script Package

(9327177D)

This document describes Unpaired Terminate Golden Voice Basic Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327177D	     Unpaired Terminate GV Basic (Analog and CAS)   	         T GV BASIC

This is an unpaired terminate Golden Voice Basic script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired originate Golden Voice Basic script (part # 9327176).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (T1), 128H (E1), 128H(Analog) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912182002C (Mu-Law) or 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for ring and answer the call.  When the call is answered a Golden Voice Tone will be sent to the originating call program.  Next this call program will measure a Golden Voice Tone sent from the originate call program.  When the call passes, a set of statistics is gathered and posted which may be viewed from the Term GV Basic report page.  If a failure occurs any time during the call, a terminate attempt statistic will be incremented and a real time error will be displayed.   The terminate line will go ONHOOK after the tone has been measured.  After completion of the call the whole process starts again.

Unpaired Golden Voice Basic Terminate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the called channel. |
| answer_delay | 0 | Delay, in seconds, before sending ANM (Answer Message). Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to sending the GV tone. |
| dly_b4_meas | 8 | Amount of time, in seconds, to delay before starting the measure. Note: The expected Sendtone must start before the measure. |
| guard_time | 1 | The number of seconds that the line will ignore any incoming calls, before it checks the next incoming call. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| snr_thresh | 28 | Signal To Noise Ratio Threshold. This call program will report a real time error for any SNR value below this threshold. |
| tone_on_time | 15 | The amount of time in seconds, that the golden voice tone will be on. |

Unpaired Terminate Golden Voice Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 11 R_TERMATTMPT | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 R_TERMCOMPL | TERM COMPL | The number of calls that the called line answered an incoming call and verified the receipt of a fax. This code counts the number of calls successfully answered by the specified terminate line. |
| 6001 R_TOTNRGMIN | MIN. TOTAL ENERGY (-) | Reports the minimum total energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Minimum Total Energy (-)’. |
| 6002 R_TOTNRGMAX | MAX. TOTAL ENERGY (-) | Reports the maximum total energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Maximum Total Energy (-)’. |
| 6003 R_SNRMIN | MIN. SIGNAL TO NOISE RATIO | Reports the minimum Signal To Noise Ration (in dB) of the GV tone in the GV Basic Term. Extended report page. |
| 6004 R_SNRMAX | MAX. SIGNAL TO NOISE RATIO | Reports the maximum Signal To Noise Ration (in dB) of the GV tone in the GV Basic Term. Extended report page. |
| 6005 R_GVNRGMIN | MIN GOLDEN VOICE ENERGY (-) | Reports the minimum golden voice energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Minimum GV Energy (-)’. |
| 6006 R_GVNRGMAX | MAX. GOLDEN VOICE ENERGY (-) | Reports the maximum golden voice energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Maximum GV Energy (-)’. |
| 6007 R_SPRNRGMAX | MAX. SPURIOUS ENERGY (-) | Reports the maximum spurious energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Maximum Spurious Energy (-)’. |
| 7025 R_SPUROVER | SPURIOUS ENERGY OVERFLOW | Reports a real-time error that the spurious energy is too high. |
| 7026 R_LOWSNR | LOW SIGNAL TO NOISE RATIO | Reports a real-time error that the Signal to Noise Ratio is too low, including the value of the low SNR, if the Signal to Noise Ratio Threshold (snr_thresh) is not met. |