CRS & AM2 SCRIPT MANUAL

For Unpaired Originate GoldenMOS (CAS) Script Package

(9327213J)

This document describes Unpaired Originate GoldenMOS Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327213J	     	Unpaired Originate GoldenMOS (CAS)            	      CAS O GMOS

This is an unpaired originate GoldenMOS script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the Analog and CAS Unpaired Terminate GoldenMOS script (part # 9327214).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128F or higher, LGC PCB part number 280277AY PCB  or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912181002B (Mu-Law) or 912032102B (A-Law)

- MMP LDSP Application 408 or higher.

- MMP DSP Application version 1Z or higher (280315/316 PCB).

- MMP power supply for units that use 280315/316 DSP boards (Digital units only).

- GMOS License Keys for the unit.

This Script will work with protocols for the following systems:

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for dial tone or seizure on the originating side and dial upon detection.  When the call is answered this call program will wait for Golden Voice ID tones.  Then the call program will wait for a Golden Voice round trip delay tone sent from the terminate call program, and responds with another Golden Voice tone back to the terminating call program.  Next, this call program sends a Golden Voice pulse to the terminate call program for it to measure the one-way delay.  It then sends a Golden Voice tone to the terminate call program for “tone_on_time” (default 20 seconds) so the terminate call program can measure the dropouts, and the energy values. If the call passed, the originate attempt and completion statistics are incremented. If a failure occurs any time during the call, an originate attempt statistic will be incremented and a real time error will be displayed.  In either case, the originate line will go ONHOOK.  After completion of the call the whole process starts again.

CAS Unpaired GoldenMOS Originate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the calling channel. |
| digits_1 |  | The phone number that the channel will dial to ring the called line. |
| digits_2 |  | Used when more than the 15 digits of digits_1are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. |
| digits_3 |  | Used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1, 4=MFR2. |
| cut_thru | 2.0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to initiating the wait for the GV tone for the round trip delay |
| delay_time | 3 | Amount of time, in seconds between GoldenMOS tests to allow the originating and terminating channels to re sync. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| intercall | 4 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| oneway_time | 4 | The amount of time the call program waits before sending the one-way pulse tone to the terminate call program. |
| RecvIDTimeout | 5 | Number of seconds allowed for the voice path confirmation (“ThreeDigitID”) before aborting the call and incrementing the “Receive ID Test Failed” counter. If the confirmation is received in the confirm time, the channel performs the remaining GoldenMOS tests and goes on-hook, delays for the time specified by the intercall parameter, then makes another call. If the time is exceeded, the call is aborted, counter (Receive ID Test Failed) is incremented, and a "1006 Receive ID Test Failed" message is displayed. (If the ThreeDigitID is never confirmed, a "1007 Receive ID Not Processed" message is displayed and the call is aborted.) |
| rtrip_timeout | 6.0 | Number of seconds to wait for the GV tone sent from the other channel during a round trip delay test |
| rtrip_tone | 14 | The GV tone to send to the other channel during a round trip delay test. Should be a GV tone only. |
| spulse_tone | 14 | The GV tone to send to the terminating channel during a receive one way delay test. Should be a GV tone only. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| ThreeDigitID | 172 | This parameter determines the sequence of Golden Voice ID tones that will be sent by the termination script to verify the voice path. The id parameter specified in the origination script must match this id to complete the voice path verification. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| tone_on_time | 20 | The amount of time in seconds that the golden voice tone will be on. |

CAS Unpaired Originate GoldenMOS Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 R_ATTEMPT | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). It counts each occurrence of an OFFHOOK instruction by the originating line regardless of the success of the call. |
| 2 R_COMPLETE | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete. It counts the number of times the programmed sequence was successful in running through its complete test. If this does not increment once per ORIG ATTEMPT, you may see any of the following codes. The code received will help you identify the fault. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 1006 R_RCVID_FAIL | Receive ID Test Failed | The number of calls that the golden voice ID path verification check failed. |
| 1007 R_RCVID_NOTP | Receive ID Not Processed | The originating channel did not receive the confirmation that the golden voice ID tones were received correctly. |
| 2001 R_S1WDLY_NOTP | Send 1-Way Delay Not Processed | The originating channel did not receive the confirmation that the one-way pulse was sent correctly. |
| 2004 R_RRTRIP_FAIL | Recv Round Trip Delay Failed | The originating channel did not receive the round trip pulse correctly. |
| 4005 R_REORDER | Reorder Detected | The number of calls that the fast busy signal was detected by the calling line, after dialing all digits. The fast busy call progress tone is characterized by a 150 to 350 ms on and off time. |
| 4006 R_LINE_BUSY | Line Busy Detected | The number of calls that the slow busy signal was detected by the calling line, after dialing all digits. The slow busy call progress tone is characterized by a 350 to 650 ms on and off time. |