CRS & AM2 SCRIPT MANUAL

For Unpaired Originate Golden Voice Diagnostic (Analog and CAS) Script Package

(9327188E)

This document describes Unpaired Originate Golden Voice Diagnostic Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327188E	     Unpaired Originate GV Diagnostic (Analog and CAS)        O GV DIAG

This is an unpaired originate Golden Voice Diagnostic script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired terminate Golden Voice Diagnostic script (part # 9327189).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (T1), 128H (E1), 128H(Analog) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912182002C (Mu-Law) or 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for dial tone or seizure on the originating side and dial upon detection.    When the call is answered this call program will wait to measure a Golden Voice tone sent from the terminate call program.  When the tone is received, the originating call program measures the tone.  If spurious energy, greater than –30dB, exists on the line, the originate call program will report the spurious energy’s centered frequency in a statistical bin.  This call program also reports the total energy of the spurious energy.  If a failure occurs any time during the call, an originate attempt statistic will be incremented and a real time error will be displayed.  If the call passes the originate line will go ONHOOK.  After completion of the call  the whole process starts again.

Unpaired Golden Voice Diagnostic Originate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the calling channel. |
| digits_1 |  | The phone number that the channel will dial to ring the called line. |
| digits_2 |  | Used when more than the 15 digits of digits_1are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. |
| digits_3 |  | Used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1, 4=MFR2. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| dly_b4_meas | 7 | Amount of time, in seconds, to delay before starting the measure. Note: The expected Sendtone must start before the measure. |
| guard_time | 6 | The number of seconds that the line will delay to allow the terminate line to complete the sending of the Golden Voice tone. |
| gv_tone | 0 | Selects the Golden Voice tone that will be sent. 0 = GV tone 29, which has frequencies centered at 1120 Hz & 1600 Hz. 1 = GV tone 30, which has frequencies centered at 2080 Hz & 2560 Hz. |
| intercall | 4 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| spur_thresh | 40 | Spurious Energy Threshold. Level, in dB, a negative value. If spurious energy (noise) is greater than this threshold then the originate call program will report the spurious energy’s centered frequency in a statistical bin. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |

Unpaired Originate Golden Voice Diagnostic Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 R_ATTEMPT | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). It counts each occurrence of an OFFHOOK instruction by the originating line regardless of the success of the call. |
| 2 R_COMPLETE | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete. It counts the number of times the programmed sequence was successful in running through its complete test. If this does not increment once per ORIG ATTEMPT, you may see any of the following codes. The code received will help you identify the fault. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 7 | AVG PD DELAY | The average time from end of dialing until the first ring is detected on the called line. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits. |
| 6001 R_TOTNRGMIN | MIN. TOTAL ENERGY (-) | Reports the minimum total energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Minimum Total Energy (-)’. |
| 6002 R_TOTNRGMAX | MAX. TOTAL ENERGY (-) | Reports the maximum total energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Maximum Total Energy (-)’. |
| 7000 R_160HZ | Non signal energy centered at 160Hz. | Reports the non-signal energy centered at 160 Hz in the GV Diag. Orig. Extended report page. |
| 7001 R_320HZ | Non signal energy centered at 320Hz. | Reports the non-signal energy centered at 320 Hz in the GV Diag. Orig. Extended report page. |
| 7002 R_480HZ | Non signal energy centered at 480Hz. | Reports the non-signal energy centered at 480 Hz in the GV Diag. Orig. Extended report page. |
| 7003 R_640HZ | Non signal energy centered at 640Hz. | Reports the non-signal energy centered at 640 Hz in the GV Diag. Orig. Extended report page. |
| 7004 R_800HZ | Non signal energy centered at 800Hz. | Reports the non-signal energy centered at 800 Hz in the GV Diag. Orig. Extended report page. |
| 7005 R_960HZ | Non signal energy centered at 960Hz. | Reports the non-signal energy centered at 960 Hz in the GV Diag. Orig. Extended report page. |
| 7006 R_1120HZ | Non signal energy centered at 1120Hz. | Reports the non-signal energy centered at 1120 Hz in the GV Diag. Orig. Extended report page. |
| 7007 R_1280 | Non signal energy centered at 1280Hz. | Reports the non-signal energy centered at 1280 Hz in the GV Diag. Orig. Extended report page. |
| 7008 R_1440HZ | Non signal energy centered at 1440Hz. | Reports the non-signal energy centered at 1440 Hz in the GV Diag. Orig. Extended report page. |
| 7009 R_1600HZ | Non signal energy centered at 1600Hz. | Reports the non-signal energy centered at 1600 Hz in the GV Diag. Orig. Extended report page. |
| 7010 R_1760HZ | Non signal energy centered at 1760Hz. | Reports the non-signal energy centered at 1760 Hz in the GV Diag. Orig. Extended report page. |
| 7011 R_1920HZ | Non signal energy centered at 1920Hz. | Reports the non-signal energy centered at 1920 Hz in the GV Diag. Orig. Extended report page. |
| 7012 R_2080HZ | Non signal energy centered at 2080Hz. | Reports the non-signal energy centered at 2080 Hz in the GV Diag. Orig. Extended report page. |
| 7013 R_2240HZ | Non signal energy centered at 2240Hz. | Reports the non-signal energy centered at 2240 Hz in the GV Diag. Orig. Extended report page. |
| 7014 R_2400HZ | Non signal energy centered at 2400Hz. | Reports the non-signal energy centered at 2400 Hz in the GV Diag. Orig. Extended report page. |
| 7015 R_2560HZ | Non signal energy centered at 2560Hz. | Reports the non-signal energy centered at 2560 Hz in the GV Diag. Orig. Extended report page. |
| 7016 R_2720HZ | Non signal energy centered at 2720Hz. | Reports the non-signal energy centered at 2720 Hz in the GV Diag. Orig. Extended report page. |
| 7017 R_2880HZ | Non signal energy centered at 2880Hz. | Reports the non-signal energy centered at 2880 Hz in the GV Diag. Orig. Extended report page. |
| 7018 R_3040HZ | Non signal energy centered at 3040Hz. | Reports the non-signal energy centered at 3040 Hz in the GV Diag. Orig. Extended report page. |
| 7019 R_3200HZ | Non signal energy centered at 3200Hz. | Reports the non-signal energy centered at 3200 Hz in the GV Diag. Orig. Extended report page. |
| 7020 R_3360HZ | Non signal energy centered at 3360Hz. | Reports the non-signal energy centered at 3360 Hz in the GV Diag. Orig. Extended report page. |
| 7021 R_3520HZ | Non signal energy centered at 3520Hz. | Reports the non-signal energy centered at 3520 Hz in the GV Diag. Orig. Extended report page. |
| 7022 R_3680 | Non signal energy centered at 3680Hz. | Reports the non-signal energy centered at 3680 Hz in the GV Diag. Orig. Extended report page. |
| 7023 R_3840HZ | Non signal energy centered at 3840Hz. | Reports the non-signal energy centered at 3840 Hz in the GV Diag. Orig. Extended report page. |
| 7024 R_NONRG | Failed to detect energy | Counts the total number of times the spurious energy was greater than –30dB, and mis-detected the centered frequency. |