CRS & AM2 SCRIPT MANUAL

For Unpaired Originate Golden Voice ID (Analog and CAS) Script Package

(9327194B)

This document describes Unpaired Originate Golden Voice ID Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327194B	          Unpaired Originate GV ID (Analog and CAS)   	         O GV ID

This is an unpaired originate Golden Voice ID script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired terminate Golden Voice ID script (part # 9327195).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (T1), 128H (E1), 128H(Analog) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912182002C (Mu-Law) or 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for dial tone or seizure on the originating side and dial upon detection.  When the call is answered, the Golden Voice SENDID/RECVID tone sequence will be used for voice path verification. Conversation time is controlled by the originating side. Failure to verify voice path will end the test sequence.  The terminate line will go ONHOOK after receiving the Golden Voice ID tone from the originate.  After completion of the call the whole process starts again.

Unpaired Golden Voice ID Originate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the calling channel. |
| digits_1 |  | The phone number that the channel will dial to ring the called line. |
| digits_2 |  | Used when more than the 15 digits of digits_1are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. |
| digits_3 |  | Used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1, 4=MFR2. |
| answer_y-1 | 0 | Whether or not reverse-battery answer supervision is required in the voice path confirmation check sequence (0=not required, 1=required). |
| confirm_time | 10 | Number of seconds allowed for the voice path confirmation (“id” and, if selected, “answer_y-1”) from the beginning of “cut_thru”, before aborting the call and incrementing the “no confirm” counter. If the confirmation is received in the confirm time, the channel goes on-hook, delays for the time specified by the intercall parameter, then makes another call. If the time is exceeded, the call is aborted, counter (Code 8) is incremented, and a "008 Confirming Failure" message (including time for answer signal) is logged. (If the id is never confirmed, a "255 System Error 3" message is logged and the call is aborted.) |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| guard_time | 4 | The number of seconds that the line will delay to allow the terminate line to complete the sending of the Golden Voice ID tones. |
| talk_time | 0 | Amount of time, in seconds, to hold up a line that is in a conversation mode (called line has answered and the voice path has been confirmed) before terminating the call (both lines going on hook). |
| id | 172 | This parameter determines the sequence of Golden Voice ID tones that will be sent by the termination script to verify the voice path. The id parameter specified in the origination script must match this id to complete the voice path verification. |
| intercall | 4 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| tone_on_time | 15 | The amount of time in seconds, that the golden voice tone will be on. |

Unpaired Originate Golden Voice ID Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 R_ATTEMPT | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). It counts each occurrence of an OFFHOOK instruction by the originating line regardless of the success of the call. |
| 2 R_COMPLETE | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete. It counts the number of times the programmed sequence was successful in running through its complete test. If this does not increment once per ORIG ATTEMPT, you may see any of the following codes. The code received will help you identify the fault. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 7 | AVG PD DELAY | The average time from end of dialing until the first ring is detected on the called line. |
| 8 | NO CONFIRM | The number of calls that the voice path verification check failed. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits. |