AM2/CRS-A/D(e) SCRIPT MANUAL

for Analog/CAS A Calls B, B Calls A VoP 1-Way Delay Script

(9327086B)

This document describes a script developed for the AM2/CRS-A/D(e) Bulk Call Generator. It consists of five parts.

| Part 1: A title and description of the script | ---Page 1 |
| --- | --- |
| Part 2: Call Flow | ---Page 2 |
| Part 3: Description of Script Parameters | ---Page 2,3,4 |
| Part 4: Explanation of Report Codes | ---Page 5 |
| Part 5: 1-Way Delay Test Specific Report Codes | ---Page 5 |

Script Number	      	      Title Description			AM2 & CRS Selection

9327086B	      A > B,B>A VoP 1-Way Delay	                          A>B;B>A 1WAY

WARNING:  This script is developed to be used on Analog, T1/E1 CAS units equipped with 25-0419 VoP Option.

REQUIREMENTS:

- Host Software Version 4.77a or higher.

- 25-0419-A/D(e) Option.

CALL FLOW

The call program will wait for dial tone or seizure on the originating side and dial upon detection.   The terminating side will wait for ringing and goes OFFHOOK when the set number of rings is reached.  When the call is answered, two 1-Way Delay Tests will be performed. Both lines will go ONHOOK right after both tests are complete. The same test will be repeated in the reverse direction before the entire scenario starts over.

SCRIPT PARAMETERS

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| a_channel |  | The channel number of the calling channel when A calls B The channel number of the called channel when B calls A |
| ba_calld_no1 |  | The phone number that the a_channel will dial to ring the b_channel. |
| ba_calld_no2 |  | Used when more than the 15 digits of ba_calld_no1 are required. These digits are dialed immediately after the digits programmed in the “ba_calld_no1” field. |
| ba_calld_no3 |  | Used when more than the 30 digits of ba_calld_no1 and ba_calld_no2 are required. These digits are dialed immediately after the digits programmed in the “ba_calld_no2” field. |
| answer_delay | 0.0 | Delay, in seconds, before terminating answers. Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| b_channel |  | The channel number of the calling channel when B calls A The channel number of the called channel when A calls B |
| ab_calld_no1 |  | The phone number that the b_channel will dial to ring the a_channel. |
| ab_calld_no2 |  | Used when more than the 15 digits of ab_calld_no1 are required. These digits are dialed immediately after the digits programmed in the “ab_calld_no1” field. |
| ab_calld_no3 |  | Used when more than the 30 digits of ab_calld_no1and ab_calld_no2 are required. These digits are dialed immediately after the digits programmed in the “ab_calld_no2” field. |
| call_call<s> | 0 | Time, in seconds, from the start (off hook) of one call until the start of the next call. This parameter cannot be used to gain a higher volume of calls. It is only used to “throttle back” the calling rate to a rather predictable amount (i.e. If the parameters controlling the line (and the speed of the switch under test) allowed for one completed call every six seconds, or 10 calls per minute, but, the desired rate was only four calls per minute, this parameter would be set to 15. In this manner, there would be a 15 second period between off hooks, or four calls per minute). Keep in mind that for failed calls, depending on how they fail, other timers may not allow for the line to be released (put back on hook) in time to start the next call. In this case, when the line does go on hook, it will wait the programmed intercall time and go back off hook. |
| cut_thru | 2.0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay from called line going off hook until voice path confirmation check is initiated. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1. |
| intercall | 3 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. In locations where the ringing is an interrupted ring, this time should be set to bridge all quiet periods within a single ring cycle. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| ring_timeout | 10 | Number of seconds to wait from the time that the last digit is dialed until the first ring is detected at the called line before aborting the call and incrementing the “no alert” counter. This parameter should be set at least as long as it takes the switch to process the dialed digits and ring the called line under load conditions to prevent premature aborting of a call. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| rpulse_timer | 4 | The rpulse_timer parameter selects the timeout for the receive 1-way tone pulse. |
| spulse_timer | 4 | The spulse_timer parameter selects the timeout for the send 1-way tone pulse. |
| spulse_tone | 14 | The spulse_tone parameter selects the tone that is sent during the pulse test. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| bin_1_max bin_2_max bin_3_max bin_4_max |  | The bin_max parameters are used to define the statistical bins which tally the dropout test results 1-Way Delay Tests. Each test counts the number of dropouts/lost packets detected during the dropout test stage. The number of dropout counts are calculated into the dropout average drops per call (CODE 2120). In addition, one of the statistical bins is incremented for each completed test. A count is scored in the bin in which the dropout count for the call falls. The bin dropout ranges are defined as follows: DROP_BIN_1 0 to bin1_max 0 DROP_BIN_2 (bin1_max + 5ms) to bin_2_max 01 - 50 DROP_BIN_3 (bin2_max + 5ms) to bin_3_max 51 - 100 DROP_BIN_4 (bin3_max + 5ms) to bin_4_max 101 - 250 DROP_BIN_5 (bin4_max + 5ms) and up 251 + |

REPORT CODES

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). |
| 2 | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete, including voice path verification and answer supervision check, if enabled. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 6 | NO ALERT | The number of times that a call progresses to the point that the called line expects ringing and doesn’t get it. |
| 7 | AVG PD DELAY | The average time from end of dialing until the first ring is detected on the called line. |
| 8 | NO CONFRM | The number of times the 1-Way Delay Test failed. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits and waiting the "ring_timeout" period. |
| 10 | NO ANS SIGNAL | The number of calls that reverse battery was not detected at the end of the voice path confirmation check with detection enabled. |
| 11 | TERM ATTMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. |
| 12 | TERM COMPL | The number of calls that the called line answered an incoming call and verified the voice path. |

REPORT CODES USED BY THE ONE-WAY DELAY TEST

2100 AVER_DELAY: Average 1-Way Delay. Reports the average one-way delay in milliseconds.

2101 MIN_DELAY: Minimum 1-Way Delay. Reports the minimum one-way delay in milliseconds.

2102 MAX_DELAY: Maximum 1-Way Delay. Reports the maximum one-way delay in milliseconds.

2103 DELAY_BIN_1: 1-Way Delay Bin 1 Counts. 100 - counts the number of calls where the one-way delay is less than 100 milliseconds.

2104 DELAY_BIN_2: 1-Way Delay Bin 2 Counts. 200 - counts the number of calls where the one-way delay is between 101 and 200 milliseconds.

2105 DELAY_BIN_3: 1-Way Delay Bin 3 Counts. 300 - counts the number of calls where the one-way delay is between 201 and 300 milliseconds.

2106 DELAY_BIN_4: 1-Way Delay Bin 4 Counts. 400 - counts the number of calls where the one-way delay is between 301 and 400 milliseconds.

2107 DELAY_BIN_5: 1-Way Delay Bin 5 Counts. Counts the number of calls where the one-way delay is greater than 401 milliseconds.