AM2/CRS-A/D(e) SCRIPT MANUAL

for Analog/CAS VoP Trip Delay Origination Script

( 9327046K )

This document describes a script developed for the AM2/CRS-A/D(e) Bulk Call Generator. It consists of four parts.

| Part 1: A title and description of the script | ---Page 1 |
| --- | --- |
| Part 2: Call Flow | ---Page 2 |
| Part 3: Description of Script Parameters | ---Page 2,3 |
| Part 4: Explanation of Report Codes | ---Page 3 |

| Script Number | Title Description | AM2-A/D(e) Selection |
| --- | --- | --- |
| #### 9327046K | Analog/CAS VoP Trip Delay Originate | ORIG Trip |

WARNING:  This script is developed to be used on Analog, T1/E1 CAS units equipped with 25-0419 VoP Option.

REQUIREMENTS:

- Host Software Version 4.77a or higher.

- 25-0419-A/D(e) Option.

CALL FLOW

# The program calls the terminating line and checks for answer supervision.  After the voice path is established the program waits for the test tone pulse coming from the terminating side. When the pulse is received a short delay is performed.  The terminating side begins looking for the return tone pulse.  After the originating side sends a return pulse, the terminating side measures and stores the round trip delay result. (Please refer to the VoP Application User`s Guide for more information) The calling line will go onhook after the testing is complete.

# SCRIPT PARAMETERS

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The analog line number of the terminating line. 1-160 A time slot in one of the four T1/E1 trunks in which voice will be sent once the call is connected. The acceptable range is A01 to D24 for a T1 unit and A01 to D31 on an E1 unit. A01 - D24, A01 - D31 |
| dial_type |  | Dial type. Types available: 1=Pulse , 2=DTMF, 3=MFR1, 4=MFR2. Normally keep this value at 2 for Analog/CAS Signaling. |
| digits_1 digits_2 digits_3 |  | The phone number that the channel will dial to ring the called line. digits_2 is used when more than the 15 digits of digits_1 are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. digits_3 is used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| answer_timer | 5.0 | Timeout value for answer supervision. |
| dial_delay | 0 | Number of seconds delay from the time dial tone is detected until start of dialing. |
| intercall | 4 | Time, in seconds, between the end of one call and the beginning of the next call. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_hi | 0 | Same as offset_lo, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_lo | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| rtrip_tone | 14 | The rtrip_tone parameter selects the tone that is sent as the return pulse during the trip test. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| rtrip_timeout | 5.0 | The rtrip_timeout parameter selects the timeout for the receive trip on the originating side. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to initate a call until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t initiate at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, the timed_start will only delay before the initial off hook, from issuance of a “start” command. |

REPORT CODES

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| R_ATTEMPT (CODE 1) | Origination Attempts | Counts each occurrence of an OFFHOOK occurrence regardless of the success of the call. |
| R_COMPLETE (CODE 2) | Origination Completes | Counts the number of times the programmed sequence was successful in running through its complete test. |
| R_STARTDELAY (CODE 3) | Average Start Delay | Average time it takes to receive dial tone. |
| R_SLOWSTART (CODE 4) | Slow Start Signal x | The expected signal to start dialing was not detected by the protocol within the delay timeout set by the 'st_sig_dly' parameter. |
| R_NOSTART (CODE 5) | No Start Signal x | The expected signal to start dialing was not detected by the protocol within the fail timeout set by the 'st_sig_fail' parameter. |
| R_PDTIME (CODE 7) | Avg PD Delay | Average Post Dial Delay. |
| R_NOCONFIRM (CODE 8) | No Confirm x | Number of times round trip delay test times out. |

Note: See your instruction manual or the Script Writing Guide for an explanation of other report codes which may occur.