This document describes a script developed for the AM2/CRS-A and the AM2/CRS-D Bulk Call Generators. It consists of four parts.

| Part 1: Title and Description of the script | --- Page 1 |
| --- | --- |
| Part 2: Script Parameters | --- Page 1 – 2 |
| Part 3: Report Codes | --- Page 2 |
| Part 4: Standard Codes | --- Page 3 |

| Script Number | Title Description | ## AM2/CRS-De Selection |
| --- | --- | --- |
| # 9327131B | Analog/CAS Unpaired Originate VOP II Dropout Call Script | # O VOP2 DROP |

This is an originating VOP script that was developed to measure dropouts.  This script will initiate a call and wait for an incoming modulated tone and analyze the tone for dropouts for the test_time parameter.  It is to be used with the VOPII DROP T script part number 9327132 or R2T VOP2 DRP

REQUIREMENT (S):

- Host version 4.77A or higher

- FeatureCall Version 3.6 or higher

SCRIPT PARAMETERS

| PARAMETER NAME | ### RANGE | ### DEFINITION |
| --- | --- | --- |
| channel | (Analog)1-64 or 128 (Digital) A1 to D24 or A1 to D31 | The channel number of the calling line. |
| dial_type | 1-3 | Type of dialing used to begin dialing. 1=Dial pulse, 2=DTMF, 3=MFR1. |
| digits_1 | 0-9, A-D, *, #, f, p, w, t | The phone number that the a_channel will dial to ring the b_channel. See below for ranges and definitions of the digits “f, p, w, and t”. |
| digits_2 | Same as digits_1 | Used when more than the 15 digits of digits_1 are required. These digits are dialed immediately after the digits programmed into the “digits_1” field. |
| digits_3 | Same as digits_1 | Used when more than 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| “f” (followed by a number) | 1-9, 0 | Number of hundred_milliseconds to be on hook for a “hook flash” (1=100ms, 2=200ms…0=1000ms) |
| “p” (followed by a number) | 1-9, 0 | Number of seconds to pause for dial tone (1=1 sec, 2=2 sec…0=10 sec.) |
| “w” (followed by a number) | 1-9, 0 | Number of seconds to wait before continuing the dialing string (1=1 sec, 2=2 sec…0=10 sec) |
| “t” (followed by a number) | 1-3 | Direct selection of a dialing type other than that selected. 1 = Dial Pulse, 2 = DTMF, 3 = MF R1 (North America Type) |
| answer_timer | 0-6553 | Time in seconds to wait for an answer signal. If an answer signal is not detected it then checks for a busy signal. If a busy signal is not detected the call continuous as programmed. |
| cut_thru | 0-6553 | Number of seconds delay from calling line going off hook until voice path confirmation is initiated. |
| det_lost_sig | 0, 1 | Selects whether a loss of test signal will result in a failed call 200ms after the signal is lost. If the tone is to failed upon the loss of the test signal (e.g., loss of voice path), det_lost_sig should be set to 1. If the call should not be dropped set det_lost_sig to 0. In this case, if the signal returns, dropout detection begins again when the signal is detected. |
| dial_delay | 0-6553 | Number of seconds to delay from the time dial tone is detected until start of dialing. |
| drop_bin1 |  | Sets the upper limit range of BIN 1 in milliseconds. Each BIN is incremented for each dropout detected. A count is scored in the bin in which the dropout duration falls. The default range is 1 to 10 milliseconds. |
| drop_bin2 |  | Sets the upper limit range of BIN 2 to BIN 1 in milliseconds. Default range is 11 to 20 milliseconds. |
| drop_bin3 |  | Sets the upper limit range of BIN 3 to BIN 2 in milliseconds. Default range is 21 to 50 milliseconds. |
| drop_bin4 |  | Sets the upper limit range of BIN 4 to BIN 3 in milliseconds. Default range is 50 to 100 milliseconds. Note: The range for DROP_BIN5 is 101 milliseconds and greater. |
| intercall | 0-6553 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_l | 0 to –30 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated, Typical telephones are fixed to send –9dB. |
| level_h | 0 to –30 | Same as level_l, but for the high band. The difference between the setting in level_l and level_h, if any, is referred to as the amount of “twist”, expressed in dB. |
| offset_l | -9.9 to 9.9 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| offset_h | -9.9 to 9.9 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| ontime | 5-1275 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| offtime | 5-1275 | The amount of time, in milliseconds, of quite time following the “ontime” of each tone dialed digit. |
| st_sig_dly | 0-6553 | Number of seconds to wait from off hook on calling line until dial tone is detected before that line’s call statistics increment the “slow start” counter. Used to track the number of slow dial tones. |
| st_sig_fail | 0-6553 | Number of seconds to wait from off hook on calling line before aborting this attempted call if no dial tone is detected. If this happens, the call statistics counter, “no start” will be incremented. |
| test_time | Max.=3276 | The time in seconds that dropouts are to be measured. This parameter should be 5 seconds lower than the terminating Call Program’s test_time parameter. |
| timed_start | 0-99 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all the originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| vop_timeout | 0-6553 | The time in seconds that the originating side will wait for an incoming modulated tone before failing the call. |

# REPORT CODES

| ### CALL STATISTICS | CODE | ### DEFINITION |
| --- | --- | --- |
| AVER_DROP_DUR | 2000 | Average Dropout Duration. A running average of the dropout duration in milliseconds. |
| MAX_DROP_DUR | 2001 | Maximum Dropout Duration. Reports the maximum dropout duration in milliseconds. |
| DROPOUT_BIN1 | 2002 | Dropout Bin 1 Counts. Counts the number of times the signal was not detected in the range of the drop_bin1 parameter. Default range is 1 to 10 milliseconds. |
| DROPOUT_BIN2 | 2003 | Dropout Bin 2 Counts. Counts the number of times the signal was not detected in the range of the drop_bin2 parameter. Default range is 11 to 20 milliseconds. |
| DROPOUT_BIN3 | 2004 | Dropout Bin 3 Counts. Counts the number of time the signal was not detected in the range of the drop_bin3 parameter. Default range is 21 to 50 milliseconds. |
| DROPOUT_BIN4 | 2005 | Dropout BIN 4 Counts. Counts the number of times the signal was not detected in the range of the drop_bin4 parameter. Default range is 51 to 100 milliseconds. |
| DROPOUT_BIN5 | 2006 | Dropout Bin 5 Counts. Counts the number of times the signal was not detected in the range of the drop_bin1 parameter. Default range is greater than 100 milliseconds. |
| No Tone Detected | 8001 | Counts the number of times the call failed due to the incoming tone not being detected by the vop_timeout. |
| Tone Lost | 8004 | Counts the number of times the call failed due to losing the incoming tone before the test_time parameter expires. |

# STANDARD CODES

Note:  Standard codes must be added to the VOPII DROP O report page using the report format menu.

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| R_ATTEMPT (CODE 1) | Origination Attempts | Counts each occurrence of an OFFHOOK occurrence regardless of the success of the call. |
| R_COMPLETE (CODE 2) | Origination Completes | Counts the number of times the programmed sequence was successful in running through its complete test. |
| R_STARTDELAY (CODE 3) | Average Start Delay | Average time it takes to receive dial tone. |
| R_SLOWSTART (CODE 4) | Slow Start Signal x | The expected signal to start dialing was not detected by the protocol within the delay timeout set by the 'st_sig_dly' parameter. |
| R_NOSTART (CODE 5) | No Start Signal x | The expected signal to start dialing was not detected by the protocol within the fail timeout set by the 'st_sig_fail' parameter. |
| R_PDTIME (CODE 7) | Avg PD Delay | Average Post Dial Delay. |
| R_NOCONFIRM (CODE 8) | No Confirm x | Number of times VoP Signal Analysis times out. |
| SYSTEM ERROR (CODE 255) | System Error x | System error failure due to result collection failure. |