AM2/CRS-A/D(e) SCRIPT MANUAL

for Analog/CAS VoP Detection Termination Script

(9327155B)

This document describes a script developed for the AM2/CRS-A/D(e) Bulk Call Generator. It consists of six parts.

| Part 1: A title and description of the script | ---Page 1 |
| --- | --- |
| Part 2: Call Flow | ---Page 2 |
| Part 3: Programming Notes | ---Page 2 |
| Part 4: Description of Script Parameters | ---Page 2,3 |
| Part 5: Explanation of Report Codes | ---Page 3 |
| Part 6: Report codes used by the One-way Delay test script | ---Page 3 |

| Script Number | Title Description | AM2-A/D(e) Selection |
| --- | --- | --- |
| #### 9327155B | Analog/CAS VoP Detection Terminate | VoP DT C A T |

WARNING:  This script is developed to be used on Analog, T1/E1 CAS units equipped with 25-0419 VoP Option.   This script must be used with 9327154C VoP DT C A O.

This script uses DTMF digits to transmit information about the type of the test to be performed.  Some

Codecs may not pass the DTMF digits through the network or may corrupt them causing call failures.

This Script can only be used on networks that will pass these DTMF digits through such as G.711.

REQUIREMENTS:

- Host Software Version 4.83a or higher.

- 25-0419-A/D(e) Option.

CALL FLOW

# After the terminate side goes offhook and starts waiting for the digit coming from the originate side in order to proceed to the proper testing selection.  Timeout value is set by digittimeout and is defaulted to 10 seconds.  The call will be released upon failure receiving the test mode digit.  After receiving the expected digit the script will either send a VoP tone for the originate side to run a VoP Signal Analysis or it will perform a two way one-way delay test.

# PROGRAMMING NOTES

Increase digittimeout value if the terminating side does not receive the digit on time due to long call setup times.

# SCRIPT PARAMETERS

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The analog line number of the terminating line. 1-160 A time slot in one of the four T1/E1 trunks in which voice will be sent once the call is connected. The acceptable range is A01 to D24 for a T1 unit and A01 to D31 on an E1 unit. A01 - D24, A01 - D31 |
| answer_delay | 1.0 | Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| bin_1_max bin_2_max bin_3_max bin_4_max |  | The bin_max parameters are used to define the statistical bins which tally the dropout test results 1-Way Delay Tests. Each test counts the number of dropouts/lost packets detected during the dropout test stage. The number of dropout counts are calculated into the dropout average drops per call (CODE 2120). In addition, one of the statistical bins is incremented for each completed test. A count is scored in the bin in which the dropout count for the call falls. The bin dropout ranges are defined as follows: DROP_BIN_1 0 to bin1_max 0 DROP_BIN_2 (bin1_max + 5ms) to bin_2_max 01 - 50 DROP_BIN_3 (bin2_max + 5ms) to bin_3_max 51 - 100 DROP_BIN_4 (bin3_max + 5ms) to bin_4_max 101 - 250 DROP_BIN_5 (bin4_max + 5ms) and up 251 + |
| cut_thru | 2.0 | Time in seconds from the time the terminate side receives the test selection digits and actually starts performing the selected test mode. |
| digittimeout | 10 | Timeout value in seconds the terminate side will wait for the digit coming from the originate side to switch to the proper test mode. |
| duty_cycle | 50 | The duty_cycle parameter selects the percentage of the VoP Signal Analysis Test period that is to be transmitting the signal. The parameter should be entered as a percentage. The following values are allowed: 25, 50, 75 and 100%. If 100% is selected, only dropouts are measured during the analysis period (no clipping or jitter). For unpaired calls, the duty_cycle must be the same on both the originating and terminating side of the call. |
| guard_time | 0 | The number of seconds that the line will ignore any incoming calls, before it checks for the next incoming call. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. In locations where the ringing is an interrupted ring, this time should be set to bridge all quiet periods within a single ring cycle. |
| rpulse_timer | 8 | The rpulse_timer parameter selects the timeout for the receive 1-way tone pulse. |
| spulse_tone | 14 | The spulse_tone parameter selects the tone that is sent during the pulse test. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| test_time | 35 | The test_time parameter selects the duration of time that dropouts, clipping, and jitter are to be measured. For unpaired calls, test_time needs to be set 5 seconds lower than the terminating Call Program's test_time, to allow the analysis to complete before ending the tone. In the unpaired case, test_time selects the analysis duration on the originating side and the tone duration on the terminating side. |
| tone_period | 1 | The tone_period parameter selects the duration of the VoP Signal Analysis Test period. The parameter should be entered in seconds. The following values are allowed: 1,2,3 or 4. For unpaired calls, the tone_period must be the same on both the originating and terminating side of the call. |
| vop_tone | 14 | The vop_tone parameter selects the modulated tone that is sent to the detector on the far side of the call. The tone is set to the pseudo-voice tone (Tone 14) by default. |

REPORT CODES

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| R_TERMATMPT (CODE 11) | Termination Attempts | Counts each occurrence of an incoming IAM message with matching point codes, cic, and sio values regardless of the success of the call. |
| R_TERMCOMPL (CODE 12) | Termination Completes | Counts the number of times the programmed sequence was successful in running through its complete test. |
| R_NOCONFIRM (CODE 8) | No Confirm 0 | Receive Digits Timeout or Receive 1-Way Delay Failure. |
| R_NoDigitMtch (CODE 5001) | No Digits Match | No Match for Incoming Test Selection Digit |

Note: See your instruction manual or the script writing guide for an explanation of other report codes which may occur.

REPORT CODES USED BY THE ONE-WAY DELAY TEST

2100 AVER_DELAY: Average 1-Way Delay. Reports the average one-way delay in milliseconds.

2101 MIN_DELAY: Minimum 1-Way Delay. Reports the minimum one-way delay in milliseconds.

2102 MAX_DELAY: Maximum 1-Way Delay. Reports the maximum one-way delay in milliseconds.

2103 DELAY_BIN_1: 1-Way Delay Bin 1 Counts. 100 - counts the number of calls where the one-way delay is less than 100 milliseconds.

2104 DELAY_BIN_2: 1-Way Delay Bin 2 Counts. 200 - counts the number of calls where the one-way delay is between 101 and 200 milliseconds.

2105 DELAY_BIN_3: 1-Way Delay Bin 3 Counts. 300 - counts the number of calls where the one-way delay is between 201 and 300 milliseconds.

2106 DELAY_BIN_4: 1-Way Delay Bin 4 Counts. 400 - counts the number of calls where the one-way delay is between 301 and 400 milliseconds.

2107 DELAY_BIN_5: 1-Way Delay Bin 5 Counts. Counts the number of calls where the one-way delay is greater than 401 milliseconds.