AM2 Niagara, AM2 Squirt and CRS SCRIPT MANUAL

###### For A>B VoP Signal Analysis Script Package

(9327083C)

This document describes A>B VoP Signal Analysis Script for AM2 Niagara, AM2 Squirt and CRS units.  It consists of five parts.

Part 1: A title and description of the script						Page 1

Part 2: Call Flow									Page 2

Part 3: Description of Script Parameters						Page 2-6

Part 4: Explanation of Report Codes						Page 7

Part 5: Voice over Packet Specific Report Codes					Page 8

Script Number	      	   	   Title Description		AM2 & CRS Selection

9327083C		          A>B Vop Signal Analysis	 	            A->B VOP

This is a paired script designed for AM2 Niagara, AM2 Squirt and CRS units which will call from one line to another (A>B) and perform Voice over Packet Signal Anaysis.

WARNING:  This script is developed to be used on Analog, T1/E1 CAS units equipped with 25-0419 VoP Option.

REQUIREMENTS:

- Host Software Version 4.77a or higher.

- 25-0419-A/D(e) Option.

Call Flow

The call program will wait for dial tone or seizure on the originating side and dial upon detection.   The terminating side will wait for ringing and goes OFFHOOK when the set number of rings is reached.  When the call is answered, the terminating side will send the VoP tone and the originating side will run the signal anaysis test.  Both lines will go ONHOOK right after test time expires.  Once completed, the process repeats.

A Calls B Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| orig_channel |  | The number of the calling channel. |
| digits_1 |  | The phone number that the a_channel will dial to ring the b_channel. |
| digits_2 |  | Used when more than the 15 digits of b_fon_no_1are required. These digits are dialed immediately after the digits programmed in the “b_fon_no_1” field. |
| digits_3 |  | Used when more than the 30 digits of b_fon_no_1and b_fon_no_2 are required. These digits are dialed immediately after the digits programmed in the “b_fon_no_2” field. |
| answer_delay | 0.0 | Delay, in seconds, before terminating answers. Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| answer_y-1 | 0 | Whether or not reverse-battery answer supervision is required in the voice path confirmation check sequence (0=not required, 1=required). |
| term_channel |  | The number of the terminating channel. |
| cut_thru | 2.0 | Time in seconds from the time the originate side establishes voice path and starts performing VoP Signal Analysis. |
| det_lost_sig | 1 | The det_lost_sig parameter selects whether a loss of test signal will result in a failed call 200ms after the signal is lost. If the call is to be failed upon the loss of the test signal (e.g., loss of voice path), det_lost_sig should be set to 1. If the call should not be dropped, set det_lost_sig to 0. In this case, if the signal returns, dropout detection begins again when the signal is detected. |
| duty_cycle | 50 | The duty_cycle parameter selects the percentage of the VoP Signal Analysis Test period that is to be transmitting the signal. The parameter should be entered as a percentage. The following values are allowed: 25, 50, 75 and 100%. If 100% is selected, only dropouts are measured during the analysis period (no clipping or jitter). For unpaired calls, the duty_cycle must be the same on both the originating and terminating side of the call. |
| call_call<s> | 0 | Time, in seconds, from the start (off hook) of one call until the start of the next call. This parameter cannot be used to gain a higher volume of calls. It is only used to “throttle back” the calling rate to a rather predictable amount (i.e. If the parameters controlling the line (and the speed of the switch under test) allowed for one completed call every six seconds, or 10 calls per minute, but, the desired rate was only four calls per minute, this parameter would be set to 15. In this manner, there would be a 15 second period between off hooks, or four calls per minute). Keep in mind that for failed calls, depending on how they fail, other timers may not allow for the line to be released (put back on hook) in time to start the next call. In this case, when the line does go on hook, it will wait the programmed intercall time and go back off hook. |
| confirm_time | 5 | Number of seconds allowed for the voice path confirmation (“id” and, if selected, “answer_y-1”) from the beginning of “cut_thru”, before aborting the call and incrementing the “no confirm” counter. If the confirmation is received in the confirm time, the channel goes on-hook, delays for the time specified by the intercall parameter, then makes another call. If the time is exceeded, the call is aborted, counter (Code 8) is incremented, and a "008 Confirming Failure" message (including time for answer signal) is logged. (If the id is never confirmed, a "255 System Error 3" message is logged and the call is aborted.) |
| cut_thru | 2.0 | Amount of time, in seconds, after the terminating line answers the phone and sends the VoP tone. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1. |
| intercall | 3 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. In locations where the ringing is an interrupted ring, this time should be set to bridge all quiet periods within a single ring cycle. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| ring_timeout | 10 | Number of seconds to wait from the time that the last digit is dialed until the first ring is detected at the called line before aborting the call and incrementing the “no alert” counter. This parameter should be set at least as long as it takes the switch to process the dialed digits and ring the called line under load conditions to prevent premature aborting of a call. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| talk_time | 0 | Amount of time, in seconds, to hold up a line that is in a conversation mode (called line has answered and the voice path has been confirmed) before terminating the call (both lines going on hook). |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| test_time | 30 | The test_time parameter selects the duration of time that dropouts, clipping, and jitter are to be measured. For unpaired calls, test_time needs to be set 5 seconds lower than the terminating Call Program's test_time, to allow the analysis to complete before ending the tone. In the unpaired case, test_time selects the analysis duration on the originating side and the tone duration on the terminating side. |
| tone_period | 1 | The tone_period parameter selects the duration of the VoP Signal Analysis Test period. The parameter should be entered in seconds. The following values are allowed: 1,2,3 or 4. For unpaired calls, the tone_period must be the same on both the originating and terminating side of the call. |
| spulse_tone | 14 | The spulse_tone parameter selects the tone that is sent during the pulse test. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| vop_tone | 14 | The vop_tone parameter selects the modulated tone that is sent to the detector on the far side of the call. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| vop_timeout | 12 | The vop_timeout parameter indicates the time that the originating side will wait for an incoming modulated tone before failing the call. |
| beclip_bin1 beclip_bin2 beclip_bin3 beclip_bin4 |  | The beclip_bin parameters are used to define the statistical bins that tally the trailing-edge clipping test results. Each test counts the number of trailing-edge clips detected during the signal analysis test stage. The number of trailing-edge clips is calculated into the trailing-edge clip average (CODE 2020). In addition, one of the statistical bins is incremented for each trailing-edge clip detected. A count is scored in the bin in which the trailing-edge clip duration falls. Each bin range is defined as a range in milliseconds. The bin trailing-edge clip ranges are as follows: Bin Clip Duration in ms (0-315ms) Default BCLIP_BIN1 0 to beclip_bin1 0 - 10 BCLIP_BIN2 (beclip_bin1 + 5ms) to bclip_bin2 11 - 20 BCLIP_BIN3 (beclip_bin2 + 5ms) to bclip_bin3 21 - 50 BCLIP_BIN4 (beclip_bin3 + 5ms) to bclip_bin4 51 – 100 BCLIP_BIN5 (beclip_bin4 + 5ms) and up 101 + |
| drop_bin1 drop_bin2 drop_bin3 drop_bin4 |  | The drop_bin parameters are used to define the statistical bins that tally the dropout test results. Each test counts the number of dropouts/lost packets detected during the dropout test stage. The number of dropouts is calculated into the dropout average (CODE 2000). In addition, one of the statistical bins is incremented for each dropout detected. A count is scored in the bin in which the dropout duration falls. Each bin range is defined as a range in milliseconds. The bin dropout ranges are defined as follows: Bin Dropout Duration in Milliseconds Default DROPOUT_BIN1 0 to drop_bin1 0 - 10 DROPOUT_BIN2 (drop_bin1 + 5ms) to drop_bin2 11 - 20 DROPOUT_BIN3 (drop_bin2 + 5ms) to drop_bin3 21 - 50 DROPOUT_BIN4 (drop_bin3 + 5ms) to drop_bin4 51 – 100 DROPOUT_BIN5 (drop_bin4 + 5ms) and up 101 + |
| feclip_bin1 feclip_bin2 feclip_bin3 feclip_bin4 |  | The feclip_bin parameters are used to define the statistical bins that tally the leading-edge clipping test results. Each test counts the number of leading-edge clips detected during the signal analysis test stage. The number of leading-edge clips is calculated into the leading-edge clip average (CODE 2010). In addition, one of the statistical bins is incremented for each leading-edge clip detected. A count is scored in the bin in which the leading-edge clip duration falls. Each bin range is defined as a range in milliseconds. The bin leading-edge clip ranges are as follows: Bin Clip Duration in ms (0-315ms) Default FCLIP_BIN1 0 to feclip_bin1 0 - 10 FCLIP_BIN2 (feclip_bin1 + 5ms) to fclip_bin2 11 - 20 FCLIP_BIN3 (feclip_bin2 + 5ms) to fclip_bin3 21 - 50 FCLIP_BIN4 (feclip_bin3 + 5ms) to fclip_bin4 51 - 100 FCLIP_BIN5 (feclip_bin4 + 5ms) and up 101 + |

A Calls B Script Call Statistic Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). |
| 2 | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete, including voice path verification and answer supervision check, if enabled. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 6 | NO ALERT | The number of times that a call progresses to the point that the called line expects ringing and doesn’t get it. |
| 7 | AVG PD DELAY | The average time from end of dialing until the first ring is detected on the called line. |
| 8 | NO CONFRM | The number of calls that the voice path verification check failed. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits and waiting the "ring_timeout" period. |
| 10 | NO ANS SIGNAL | The number of calls that reverse battery was not detected at the end of the voice path confirmation check with detection enabled. |
| 11 | TERM ATTMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. |
| 12 | TERM COMPL | The number of calls that the called line answered an incoming call and verified the voice path. |

VoP ANALYSIS TEST SPECIFIC REPORT CODES

2000 AVG_DROP_DUR:  Average Dropout Duration. A running average of the dropout duration in milliseconds.

2001 MAX_DROP_DUR: Maximum Dropout Duration. Reports the maximum dropout  duration in milliseconds.

2002 DROPOUT_BIN1:  Dropout Bin 1 Counts. Counts the number of dropouts detected in the range of the drop_bin1 parameter (0 to 10ms).

2003 DROPOUT_BIN2:  Dropout Bin 2 Counts. Counts the number of dropouts detected in the range from the drop_bin1 parameter to the drop_bin2 parameter (11 to 20ms).

2004 DROPOUT_BIN3:  Dropout Bin 3 Counts. Counts the number of dropouts detected in the range from the drop_bin2 parameter to the drop_bin3 parameter (21 to 50ms).

2005 DROPOUT_BIN4: Dropout Bin 4 Counts. Counts the number of dropouts detected in the range from the drop_bin3 parameter to the drop_bin4 parameter (51 to 100ms).

2006 DROPOUT_BIN5: Dropout Bin 5 Counts. Counts the number of dropouts detected in the range from the drop_bin4 parameter to the drop_bin5 parameter (greater than 100ms).

2010 AVG_FCLIP_DUR: Average Leading-Edge Clip Duration. A running average of the leading-edge clipping duration in milliseconds.

2011 MAX_FCLIP_DUR:Maximum Leading-Edge Clip Duration. Reports the maximum leading-edge clipping duration in milliseconds.

2012 FCLIP_BIN1: Leading-Edge Clip Bin 1 Counts. Counts the number of leading edge clips detected in the range of the feclip_bin1 parameter (0to 10ms).

2013 FCLIP_BIN2: Leading-Edge Clip Bin 2 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin1 parameter to the feclip_bin2 parameter (11 to 20ms).

2014 FCLIP_BIN3: Leading-Edge Clip Bin 3 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin2 parameter to the feclip_bin3 parameter (21 to 50ms).

2015 FCLIP_BIN4: Leading-Edge Clip Bin 4 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin3 parameter to the feclip_bin4 parameter (51 to 100ms).