CRS SCRIPT MANUAL

For Paired Fax (Analog and CAS) Script Package

(9327169K)

This document describes Paired Fax Script for CRS units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327169K		 Paired A calls B Fax (Analog and CAS)   	 	A->B Fax

This is a Paired fax script, designed for CRS units equipped with Multimedia hardware. This script incorporates originate and terminate capabilities.

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.01A or higher.

- LGC software version 126F or higher, LGC hardware version B or higher (280277AY PCB).

    MMP DSP Application version 2Y or higher (280315/316 PCB).

    MMP power supply (Digital units only).

   Fax License Keys for each channel.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision D or newer.)

 Ground Start (Ameritec part number : 9221002, revision D or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision D or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision D or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision D or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision M or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision D or newer)

Call Flow

This script generates an outgoing call.  The terminating side will wait for ringing and goes OFFHOOK when the set number of rings is reached.  After a connection is established, the voice path is checked by the sending and receiving of a Fax of user selectable length.  When the fax passes, a set of statistics is gathered and posted which may be viewed from the A->B Fax report page.  If the call fails, a receive failure statistic will be incremented and a real time error will be displayed.  The call is then dropped and generates another call.

Paired A Calls B Fax Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| digits_1 |  | The phone number that the orig_channel will dial to ring the term_channel. |
| digits_2 |  | Used when more than the 15 digits of digits_1are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. |
| digits_3 |  | Used when more than the 30 digits of digits_1and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1. |
| orig_channel |  | The line or timeslot number of the calling channel. |
| term_channel |  | The line or timeslot number of the terminating channel. |
| answer_delay | 0.0 | Delay, in seconds, before terminating answers. Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| call_call<s> | 0 | Time, in seconds, from the start (off hook) of one call until the start of the next call. This parameter cannot be used to gain a higher volume of calls. It is only used to “throttle back” the calling rate to a rather predictable amount (i.e. If the parameters controlling the line (and the speed of the switch under test) allowed for one completed call every six seconds, or 10 calls per minute, but, the desired rate was only four calls per minute, this parameter would be set to 15. In this manner, there would be a 15 second period between off hooks, or four calls per minute). Keep in mind that for failed calls, depending on how they fail, other timers may not allow for the line to be released (put back on hook) in time to start the next call. In this case, when the line does go on hook, it will wait the programmed intercall time and go back off hook. |
| cut_thru | 0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to the receipt of the fax transmission. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| ecm_mode | 0 | Selects whether a fax will be sent using the Error Correction Mode. 0 = No ECM, 1 = ECM |
| fax_image | 1 | Defines the test image that will be sent or received [range 1-4]. |
| fax_path_ID | 0 | Selects the path ID value that will be sent in the fax DCS message and validated by the terminate call program. 0 = Disable Path ID check, 1-65535 = Use Path ID feature. |
| intercall | 3 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| num_pages | 1 | Defines the number of times an image will be sent in a single call [range 1-200]. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| page_timeout | 75 | Defines the maximum allowable time in seconds that the test will allow a single image to be sent to the terminator. Must be set to the same value as page_timer. |
| page_timer | 75 | Defines the maximum allowable time in seconds that the test will allow a single image to be sent to the terminator. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. |
| ring_timeout | 10 | Number of seconds to wait from the time that the last digit is dialed until the first ring is detected at the called line before aborting the call and incrementing the “no alert” counter. This parameter should be set at least as long as it takes the switch to process the dialed digits and ring the called line under load conditions to prevent premature aborting of a call. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |

Paired A calls B Fax Script Data Register Codes

| CODE | TITLE |  | DESCRIPTION |
| --- | --- | --- | --- |
| 1 R_ATTMPT | ORIG ATTEMPT |  | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). It counts each occurrence of an OFFHOOK instruction by the originating line regardless of the success of the call. |
| 2 R_COMPLETE | ORIG COMPL |  | Will increment its count, up to 50000, each time the call between the two lines is complete. It counts the number of times the programmed sequence was successful in running through its complete test. If this does not increment once per ORIG ATTEMPT, you may see any of the following codes. The code received will help you identify the fault. |
| 3 R_START | AVG START DELAY |  | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START |  | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START |  | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 406 R_NOALERT | NO ALERT | The incoming call was not detected on the terminating time slot. |  |
| 7 | AVG PD DELAY |  | The average time from end of dialing until the first ring is detected on the called line. |
| 8 R_NOCONFIRM | FAX PATH FAILURE | Counts the number of fax pages that failed to complete per channel, reports T.30 Protocol Error Code. This code reports every time an R_FAXTXERR (Send Fax Failure) or an R_FAXRXERR (Receive Fax Failure) reports, and is available on the Standard Statistics page. |  |
| 9 R_BUSY | CALLED BUSY |  | The number of calls that busy signal was detected by the calling line after dialing all digits. |
| 3000 R_TXPG_ATT | SEND FAX PAGES ATTEMPTED |  | Counts the number of fax pages the channel attempted to send per channel. |
| 3001 R_TXPG_CMP | SEND FAX PAGES COMPLETED |  | Counts the number of fax pages the channel sent per channel. |
| 3002 R_RXPG_ATT | RECEIVE FAX PAGES ATTEMPTED |  | Counts the number of fax pages the channel attempted to receive per channel. |
| 3003 R_RXPG_CMPL | RECEIVE FAX PAGES COMPLETED |  | Counts the number of fax pages the channel received per channel. |
| 3004 R_FAXTXERR | SEND FAX FAILURE |  | Counts the number of fax pages that failed to complete per channel. |
| 3005 R_FAXRXERR | RECEIVE FAX FAILURE |  | Counts the number of fax pages that failed to complete per channel. |
| 3006 R_DATASPEED1 | FAXES WITH SPEED 300 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 300 bps. |
| 3007 R_DATASPEED2 | FAXES WITH SPEED 2400 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 2400 bps. |
| 3008 R_DATASPEED3 | FAXES WITH SPEED 4800 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 4800 bps. |
| 3009 R_DATASPEED4 | FAXES WITH SPEED 7200 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 7200 bps. |
| 3010 R_DATASPEED5 | FAXES WITH SPEED 9600 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 9600 bps. |
| 3011 R_DATASPEED6 | FAXES WITH SPEED 12000 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 12000 bps. |
| 3012 R_DATASPEED7 | FAXES WITH SPEED 14400 bps |  | Statistical bin to count the number of times the channel negotiated a connection speed of 14400 bps. |
| 3014 R_DATANSDLY | Average Answer Delay (msec) |  | Calculates the average time to receive the carrier tone per channel. |
| 3015 R_DATANSMAX | Maximum Answer Delay (msec) |  | Records the maximum time to receive the carrier tone per channel. |
| 3016 R_DATESTDLY | Average Establish Delay (msec) |  | Calculates the average time to complete phase B negotiation per channel. |
| 3017 R_DATESTMAX | Maximum Establish Delay (msec) |  | Records the maximum time to complete phase B negotiation per channel. |
| 3018 R_DATRES1 | STANDARD RESOLUTION FAXES |  | Statistical bin to count the number of times the channel negotiates a standard image resolution per channel. |
| 3019 R_DATRES2 | FINE RESOLUTION FAXES |  | Statistical bin to count the number of times the channel negotiates a fine image resolution per channel. |
| 3021 R_DATNOECM | Average Image Errors without ECM (%) |  | T.30 fax algorithm allows the user to verify the accuracy of an image line by line without Error Correction Mode. Provides the average percentage of lines that have errors. |
| 3022 R_DATNOECMM | Maximum Image Errors without ECM (%) |  | T.30 fax algorithm allows the user to verify the accuracy of an image line by line without Error Correction Mode. Provides the maximum percentage of lines that have errors. |
| 3023 R_DATECM | Average Image Error with ECM (%) |  | Error Correction Mode T.30 fax algorithm allows the user to verify the accuracy of an image line by line. Provides the average percentage of lines that have errors. |
| 3024 R_DATECMMAX | Maximum Image Error with ECM (%) |  | Error Correction Mode T.30 fax algorithm allows the user to verify the accuracy of an image line by line. Provides the maximum percent of lines that have errors. |
| 3025 R_DATASPPED | Failed to Detect Modem Speed |  | Counts the total number of modem speed mis-detections per channel. |
| 3026 R_DATRES | Failed to Detect Resolution |  | Counts the total number of resolution mis-detections per channel. |
| 3027 R_DIALTOUT | Dial Timeout |  | Counts the total number of times dialing is not performed per channel. |
| 3028 R_FAXERRPG | Fax Error Page |  | This real-time error indicates during which page number a fax error occurred. |
| 3119 ERR_NUPAGS | Number of Page Mismatch |  | Counts the total number of times fax calls were completed with an incorrect number of pages. |