CRS SCRIPT MANUAL

For R2 Unpaired Terminate Fax (E1 CAS) Script Package

(9327164K)

This document describes Unpaired Terminate Fax Script for CRS units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327164K		 R2 Unpaired Terminate Fax (E1 CAS)   	 	R2 T Fax

This is an unpaired R2 terminate fax script, designed for CRS units equipped with Multimedia hardware. This script is designed to work with the unpaired originate fax script (part # 9327123).  The dial_type of the originate script should be set to 4 (R2 dialing) when using this script.

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 501A or higher.

- LGC software version 126F or higher, LGC hardware version B or higher (280277AY PCB).

    MMP DSP Application version 2Y or higher (280315/316 PCB).

    MMP power supply.

   Fax License Keys for each channel.

This Script will work with protocols for the following systems:

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision M or newer)

Call Flow

A backward signal is sent by the incoming register in response to a Forward MFR2 signal. The backward signals convey information back to the outgoing register such as status of the network and what information to send next.  After the completion of this exchange the call program will answer the call.  When the call is answered a fax will be received.  When the fax passes, a set of statistics is gathered and posted which may be viewed from the R2 Term Fax report page.  If the call fails, a receive failure statistic will be incremented and a real time error will be displayed.  The terminate line will go ONHOOK after the fax is received.  After completion of the call the whole process starts again.

R2 Terminate Fax Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the called channel. |
| mfcbwd_dig_1 |  | MFR2 backwards digits to be sent in response to received Forward digits to complete the inter-register signaling requirements of the originating call. |
| mfcbwd_dig_2 |  | Used when more than the 15 digits of mfcbwd_dig_1 are required. |
| mfcbwd_dig_3 |  | Used when more than the 30 digits of mfcbwd_dig_2 are required. |
| answer_delay | 0 | Delay, in seconds, before sending ANM (Answer Message). Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to the receipt of the fax transmission. |
| fax_path_ID | 0 | Selects the path ID value that will be sent in the fax DCS message and validated by the terminate call program. 0 = Disable Path ID check, 1-65535 = Use Path ID feature. |
| guard_time | 1 | The number of seconds that the line will ignore any incoming calls, before it checks the next incoming call. |
| level_h | -9 | Level, in dB, that the high band frequencies of MFR2 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of MF R2 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of MFR2 digits. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| page_timer | 75 | Defines the maximum allowable time in seconds that the test will allow a single image to be sent to the terminator. |

R2 Terminate Fax Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 11 R_TERMATMPT | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 R_TERMCOMPL | TERM COMPL | The number of calls that the called line answered an incoming call and verified the receipt of a fax. This code counts the number of calls successfully answered by the specified terminate line. |
| 8 R_NOCONFIRM | FAX PATH FAILURE | Counts the number of fax pages that failed to complete per channel, reports T.30 Protocol Error Code. This code reports every time an R_FAXRXERR (Receive Fax Failure) reports, and is available on the Standard Statistics page. |
| 3002 R_RXPG_ATT | RECEIVE FAX PAGES ATTEMPTED | Counts the number of fax pages the channel attempted to receive per channel. |
| 3003 R_RXPG_CMPL | RECEIVE FAX PAGES COMPLETED | Counts the number of fax pages the channel received per channel. |
| 3005 R_FAXRXERR | RECEIVE FAX FAILURE | Counts the number of fax pages that failed to complete per channel, reports T.30 Protocol Error Code. |
| 3006 R_DATASPEED1 | FAXES WITH SPEED 300 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 300 bps. |
| 3007 R_DATASPEED2 | FAXES WITH SPEED 2400 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 2400 bps. |
| 3008 R_DATASPEED3 | FAXES WITH SPEED 4800 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 4800 bps. |
| 3009 R_DATASPEED4 | FAXES WITH SPEED 7200 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 7200 bps. |
| 3010 R_DATASPEED5 | FAXES WITH SPEED 9600 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 9600 bps. |
| 3011 R_DATASPEED6 | FAXES WITH SPEED 12000 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 12000 bps. |
| 3012 R_DATASPEED7 | FAXES WITH SPEED 14400 bps | Statistical bin to count the number of times the channel negotiated a connection speed of 14400 bps. |
| 3016 R_DATESTDLY | Average Establish Delay (msec) | Calculates the average time to complete phase B negotiation per channel. |
| 3017 R_DATESTMAX | Maximum Establish Delay (msec) | Records the maximum time to complete phase B negotiation per channel. |
| 3018 R_DATRES1 | STANDARD RESOLUTION FAXES | Statistical bin to count the number of times the channel negotiates a standard image resolution per channel. |
| 3019 R_DATRES2 | FINE RESOLUTION FAXES | Statistical bin to count the number of times the channel negotiates a fine image resolution per channel. |
| 3021 R_DATNOECM | Average Image Errors without ECM (%) | T.30 fax algorithm allows the user to verify the accuracy of an image line by line without Error Correction Mode. Provides the average percentage of lines that have errors. |
| 3022 R_DATNOECMM | Maximum Image Errors without ECM (%) | T.30 fax algorithm allows the user to verify the accuracy of an image line by line without Error Correction Mode. Provides the maximum percentage of lines that have errors. |
| 3023 R_DATECM | Average Image Error with ECM (%) | Error Correction Mode T.30 fax algorithm allows the user to verify the accuracy of an image line by line. Provides the average percentage of lines that have errors. |
| 3024 R_DATECMMAX | Maximum Image Error with ECM (%) | Error Correction Mode T.30 fax algorithm allows the user to verify the accuracy of an image line by line. Provides the maximum percent of lines that have errors. |
| 3025 R_DATASPEED | Failed to Detect Modem Speed | Counts the total number of modem speed mis-detections per channel. |
| 3026 R_DATRES | Failed to Detect Resolution | Counts the total number of resolution mis-detections per channel. |
| 3028 R_FAXERRPG | Fax Error Page | This real-time error indicates during which page number a fax error occurred. |