CRS & AM2 SCRIPT MANUAL

For Paired  A to B GoldenMOS (Analog and CAS) Script Package

(9327212J)

This document describes Paired A to B GoldenMOS Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327212J	     	Paired A to B GMOS (Analog and CAS)  	    A to B GMOS

This is a paired A to B GoldenMOS script, designed for CRS & AM2 units equipped with VOP3 software. This script will originate and terminate the call.

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128F or higher, LGC PCB part number 280277AY PCB  or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912181002B (Mu-Law) or 912032102B (A-Law)

- MMP LDSP Application 408 or higher.

- MMP DSP Application version 1Z or higher (280315/316 PCB).

- MMP power supply for units that use 280315/316 DSP boards (Digital units only).

- GMOS License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This script generates an outgoing call.  The terminating side will wait for ringing and goes OFFHOOK when the set number of rings is reached.  After a connection is established, the terminate channel measures a Golden Voice tone sent by the originating channel.  Next the terminate channel does a drop test on a tone sent by the originating channel.  Then, a round trip delay test is done from the terminating channel. Finally one-way delay tests are done from the terminating to originating channels, and the originating  to terminating channels in that order. When the tests are successfully completed, attempts and completions are incremented, and a set of statistics are gathered and posted which may be viewed from the A->B GMOS report page.  If the call fails, attempts are incremented and a real time error will be displayed.  The call is then dropped and generates another call.  After completion of the call the whole process starts again.

Paired A to B GoldenMOS Basic Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| digits_1 |  | The phone number that the orig_channel will dial to ring the term_channel. |
| digits_2 |  | Used when more than the 15 digits of digits_1are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. |
| digits_3 |  | Used when more than the 30 digits of digits_1and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1. |
| orig_channel |  | The line or timeslot number of the calling channel. |
| pack_size |  | The packet size of the voice packets. Reported to the GoldenMOS application to calculate the % packet loss. Legal values are 5, 10, 15, 20, 30, 40, 50, 100ms |
| pack_size_t |  | The packet size threshold in the drop test. Must be set to the same value as the pack_size for accurate results. |
| term_channel |  | The line or timeslot number of the terminating channel. |
| answer_delay | 0.0 | Delay, in seconds, before terminating answers. Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| call_call<s> | 0 | Time, in seconds, from the start (off hook) of one call until the start of the next call. This parameter cannot be used to gain a higher volume of calls. It is only used to “throttle back” the calling rate to a rather predictable amount (i.e. If the parameters controlling the line (and the speed of the switch under test) allowed for one completed call every six seconds, or 10 calls per minute, but, the desired rate was only four calls per minute, this parameter would be set to 15. In this manner, there would be a 15 second period between off hooks, or four calls per minute). Keep in mind that for failed calls, depending on how they fail, other timers may not allow for the line to be released (put back on hook) in time to start the next call. In this case, when the line does go on hook, it will wait the programmed intercall time and go back off hook. |
| cut_thru | 4.0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to sending the initial GV tone. |
| delay_time | 3 | Amount of time, in seconds between GoldenMOS tests to allow the originating and terminating channels to re sync. |
| det_lost_sig | 0 | Flag to determine whether or not the drop test should report lost signal. Set to 1 if the report is required. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| drop_timer | 4.0 | Amount of time, in seconds to wait for the drop tone from the originating channel. Normally this should occur within a second or so. |
| intercall | 3 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_hi | 0 | Same as offset_lo, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_lo | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “on_time” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. |
| ring_timeout | 10 | Number of seconds to wait from the time that the last digit is dialed until the first ring is detected at the called line before aborting the call and incrementing the “no alert” counter. This parameter should be set at least as long as it takes the switch to process the dialed digits and ring the called line under load conditions to prevent premature aborting of a call. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| rpulse_timer | 6.0 | Number of seconds to wait for the GV tone sent from the other channel during a one-way delay test |
| rtrip_timeout | 6.0 | Number of seconds to wait for the GV tone sent from the other channel during a round trip delay test |
| rtrip_tone | 14 | The GV tone to send to the other channel during a round trip delay test. Should be a GV tone only. |
| rpulse_tone | 14 | The GV tone to send to the originating channel during a send one way delay test. Should be a GV tone only. |
| spulse_timer | 4.0 | Number of seconds to send the GV tone to the other channel during a one-way delay test |
| spulse_tone | 14 | The GV tone to send to the terminating channel during a receive one way delay test. Should be a GV tone only. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| strip_timeout | 6.0 | Number of seconds to wait for the GV tone sent from the other channel during a round trip delay test |
| strip_tone | 14 | The GV tone to send back to the other channel during a round trip delay test. Should be a GV tone only. |
| test_time | 5 | The length of time in seconds the packet drop test runs. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| tone_on_time | 2 | The amount of time in seconds that the golden voice tone will be on. |

Paired A to B GoldenMOS Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 R_ATTMPT | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). It counts each occurrence of an OFFHOOK instruction by the originating line regardless of the success of the call. |
| 2 R_COMPLETE | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete. It counts the number of times the programmed sequence was successful in running through its complete test. If this does not increment once per ORIG ATTEMPT, you may see any of the following codes. The code received will help you identify the fault. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 6 | NO ALERT | The incoming call was not detected on the terminating time slot. |
| NO_CONFIRM 8 | NO CONFIRM RECEIVED | No Confirming tone received. This Real-time error reports several different places where the confirming tone failed. The parameter for the error is one of: - Failed to receive confirmation from the terminating side that the drop test GV tone was received. - During the drop test, the Signal Lost feature was enabled (det_lost_sig was set to 1), and a dropout greater than 200 ms was detected. - The originating side round trip tone was not received at the terminating channel. - The originating channel did not receive the one-way GV tone pulse sent from the terminating channel. - The terminating channel did not receive the confirmation the pulse was sent correctly. - The terminating channel did not receive the one-way GV tone pulse sent from the originating channel. - The originating channel did not receive the confirmation the pulse was sent correctly. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits. |
| 11 R_TERMATTMPT | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 R_TERMCOMPL | TERM COMPL | The number of calls that the called line answered an incoming call. This code counts the number of calls successfully answered by the specified terminate line. |
| 2100 R_DELAY_AVER | RECEIVE ONE-WAY DELAY AVG | Reports the average receive one-way delay measured by sending the GV tone from the originating side to the terminating side of the successful calls. The value is reported in ms. |
| 2102 R_DELAY_MAX | MAX. RECEIVE ONE-WAY DELAY | Reports the maximum receive one-way delay measured by sending the GV tone from the originating side to the terminating side of the successful calls. The value is reported in ms. |
| 2200 S_DELAY_AVER | RECEIVE ONE-WAY DELAY AVG | Reports the average send one-way delay measured by sending the GV tone from the terminating side to the originating side of the successful calls. The value is reported in ms. |
| 2202 S_DELAY_MAX | MAX. RECEIVE ONE-WAY DELAY | Reports the maximum send one-way delay measured by sending the GV tone from the terminating side to the originating side of the successful calls. The value is reported in ms. |
| 3110 TRIPDLY_AVER | ROUND TRIP DELAY AVG | Reports the average round trip delay for all successful calls measured by sending the GV tone from the terminating side to the originating side of the call, and back The value is reported in ms. |
| 3112 TRIPDLY_MAX | MAX. ROUND TRIP DELAY | Reports the maximum round trip delay for all successful calls measured by sending the GV tone from the terminating side to the originating side of the call, and back The value is reported in ms. |
| 3610 TERM_CHAN | TERM. CHANNEL FLAG | Reports whether the channel the data was received on is an originating channel, or a terminating channel. A value of 0 indicates an originating channel, a value of 1 a terminating channel. Used in the GMOS report generation to suppress the reporting of data from the originating side of the paired script. It is not possible to calculate the GoldenMOS score on the originating side of the call. |
| 5520 DROP_AVER | AVERAGE COUNT OF DROPPED PACKETS | Reports the average of the number of dropped packets for all successful calls. Reported as an integer count. |
| 5522 DROP_MAX | MAXIMUM COUNT OF DROPPED PACKETS | Reports the maximum number of dropped packets seen for a single call from all successful calls. Reported as an integer count. |
| 5530 DRP_TESTIME | DROP TEST TIME | This reports the duration of the drop test. The value is reported in 100ms units. With the packet size and test time, the GMOS report can calculate the 5 packet drop value required to determine the Ie value to use for the Codec. |
| 5531 DRP_PAKSIZE | DROP TEST PACKET SIZE | The packet size set for the dropped packet test. The reported packet size is the value set in the Call Program for the pack_size and pack_size_t variables. |
| 6001 R_TOTNRGMIN | MIN. TOTAL ENERGY (-) | Reports the minimum total energy (in dB) of the GV tone in the GMOS CAS Paired Extended report page. This is a negative value, as denoted by the title ‘Minimum Total Energy (-)’. |
| 6002 R_TOTNRGMAX | MAX. TOTAL ENERGY (-) | Reports the maximum total energy (in dB) of the GV tone in the GMOS CAS Paired Extended report page. This is a negative value, as denoted by the title ‘Maximum Total Energy (-)’. |
| 6003 R_SNRAVG | AVER. SIGNAL TO NOISE RATIO | Reports the average of all measured Signal To Noise Ratios (in dB) values of all successful calls with the GV tone in the GMOS CAS Paired Extended report page. |
| 6004 R_SNRMAX | MAX. SIGNAL TO NOISE RATIO | Reports the maximum Signal To Noise Ratio (in dB) of the GV tone in the GMOS CAS Extended report page. |
| 6007 R_SPRNRGMAX | MAX. NOISE ENERGY (-) | Reports the maximum spurious noise energy (in dB) of the GV tone in the GMOS CAS Extended report page. This is a negative value, as denoted by the title ‘Circuit Noise Max (-)’. |
| 6008 R_SPRNRGAVER | AVER. NOISE ENERGY (-) | Reports the average of the measured spurious noise energy (in dB) values of the GV tone in the GMOS CAS Extended report page. This is a negative value, as denoted by the title ‘Circuit Noise Avg (-)’. |