This document describes a script developed for the AM2/CRS-A and the AM2/CRS-D Bulk Call Generators. It consists of four parts.

| Part 1: Title and Description of the script | --- Page 1 |
| --- | --- |
| Part 2: Script Parameters | --- Page 1 – 3 |
| Part 3: Report Codes | --- Page 3 |
| Part 4: Standard Codes | --- Page 3 |

| Script Number | Title Description | ## AM2/CRS-De Selection |
| --- | --- | --- |
| # 9327158A | Analog/CAS Paired A calls B VOP II Dropout Call Script | # A->B DROP2 |

This is a paired VOP script that was developed to measure dropouts.  This script will initiate a call and wait for an incoming modulated tone and analyze the tone for dropouts for the test_time parameter.

REQUIREMENT (S):

- Host version 4.77A or higher

- FeatureCall Version 3.6 or higher

SCRIPT PARAMETERS

| PARAMETER NAME | ### RANGE | ### DEFINITION |
| --- | --- | --- |
| orig_channel | (Analog)1-64 or 128 (Digital) A1 to D24 or A1 to D31 | The originating channel number of the calling line. |
| term_channel | (Analog)1-64 or 128 (Digital) A1 to D24 or A1 to D31 | The terminating channel number of the called line. |
| dial_type | 1-3 | Type of dialing used to begin dialing. 1=Dial pulse, 2=DTMF, 3=MFR1. |
| digits_1 | 0-9, A-D, *, #, f, p, w, t | The phone number that the orig_channel will dial to ring the term_channel. See below for ranges and definitions of the digits “f, p, w, and t”. |
| digits_2 | Same as digits_1 | Used when more than the 15 digits of digits_1 are required. These digits are dialed immediately after the digits programmed into the “digits_1” field. |
| digits_3 | Same as digits_1 | Used when more than 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| “f” (followed by a number) | 1-9, 0 | Number of hundred_milliseconds to be on hook for a “hook flash” (1=100ms, 2=200ms…0=1000ms) |
| “p” (followed by a number) | 1-9, 0 | Number of seconds to pause for dial tone (1=1 sec, 2=2 sec…0=10 sec.) |
| “w” (followed by a number) | 1-9, 0 | Number of seconds to wait before continuing the dialing string (1=1 sec, 2=2 sec…0=10 sec) |
| “t” (followed by a number) | 1-3 | Direct selection of a dialing type other than that selected. 1 = Dial Pulse, 2=DTMF, 3 = MF R1 (North American Type) |
| answer_delay | 0-6553.5 | Number of seconds to delay from when the called line has detected a call and met the desired number of rings until it actually goes off hook. |
| answer_timer | 0-6553.5 | Time in seconds to wait for an answer signal. If an answer signal is not detected it then checks for a busy signal. If a busy signal is not detected the call continues as programmed. |
| call_call | 0-6553.5 | Time, in seconds, from the start (off hook) of one call until the start of the next call. This parameter cannot be used to gain a higher volume of calls. It is only used to “throttle back” the calling rate to a rather predictable amount (i.e. If the parameters controlling the line (and the speed of the switch under test) allowed for one completed call every six seconds, or 10 calls per minute, but, the desired rate was only four calls per minute, this parameter would be set to 15. In this manner, there would be a 15 second period between off hooks, or four calls per minute). Keep in mind that for failed calls, depending on how they fail, other timers may not allow for the line to be released (put back on hook) in time to start the next call. In this case, when the line does go on hook, it will wait the programmed intercall time and go back off hook. |
| cut_thru | 0-6553.5 | Number of seconds delay from called line going off hook until voice path confirmation is initiated. |
| det_lost_sig | 0, 1 | Selects whether a loss of test signal will result in a failed call 200ms after the signal is lost. If the tone is to failed upon the loss of the test signal (e.g., loss of voice path), det_lost_sig should be set to 1. If the call should not be dropped set det_lost_sig to 0. In this case, if the signal returns, dropout detection begins again when the signal is detected. |
| dial_delay | 0-6553.5 | Number of seconds to delay from the time dial tone is detected until start of dialing. |
| drop_bin1 | 0-65535 | Sets the upper limit range of BIN 1 in milliseconds. Each BIN is incremented for each dropout detected. A count is scored in the bin in which the dropout duration falls. The default range is 1 to 10 milliseconds. |
| drop_bin2 | 0-65535 | Sets the upper limit range of BIN 2 to BIN 1 in milliseconds. Default range is 11 to 20 milliseconds. |
| drop_bin3 | 0-65535 | Sets the upper limit range of BIN 3 to BIN 2 in milliseconds. Default range is 21 to 50 milliseconds. |
| drop_bin4 | 0-65535 | Sets the upper limit range of BIN 4 to BIN 3 in milliseconds. Default range is 50 to 100 milliseconds. Note: The range for DROP_BIN5 is 101 milliseconds and greater. |
| guard_time | 1-9 | The number of seconds that the line will ignore any incoming calls, from the time that it goes on hook of the previous call. |
| intercall | 0-6553.5 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_l | 0 to –30 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated, Typical telephones are fixed to send –9dB. |
| level_h | 0 to –30 | Same as level_l, but for the high band. The difference between the setting in level_l and level_h, if any, is referred to as the amount of “twist”, expressed in dB. |
| offset_l | -9.9 to 9.9 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| offset_h | -9.9 to 9.9 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| ontime | 5-1275 | The amount of time, in milliseconds, that the dual tones of a DTMF or MFR1 dialed digit are present for each digit. |
| offtime | 5-1275 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| rings | 1-9 | Number of rings to be detected by the called line before it goes off hook. |
| ring_cycle | 0-99 | Number of seconds from the beginning of one ring cycle until the beginning of the next ring cycle. In locations where the ringing is an interrupted ring, this time should be set to bridge all quiet periods within a single ring cycle. |
| st_sig_dly | 0-6553.5 | Number of seconds to wait from off hook on calling line until dial tone is detected before that line’s call statistics increment the “slow start” counter. Used to track the number of slow dial tones. |
| st_sig_fail | 0-6553.5 | Number of seconds to wait from off hook on calling line before aborting this attempted call if no dial tone is detected. If this happens, the call statistics counter, “no start” will be incremented. |
| test_time | 0-6553.5 | The time in seconds that dropouts are to be measured. |
| timed_start | 0-6553.5 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all the originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| vop_timeout | 0-6553.5 | The time in seconds that the originating side will wait for an incoming modulated tone before failing the call. |
| vop_tone | See Tone Table in Script Writers Guide | The modulated tone that is sent to the detector on the far side of the call. The tone is set to the pseudo-voice tone (Tone 14) by default. |

# EXTENDED REPORT CODES

| ### CALL STATISTICS | CODE | ### DEFINITION |
| --- | --- | --- |
| AVER_DROP_DUR | 2000 | Average Dropout Duration. A running average of the dropout duration in milliseconds. |
| MAX_DROP_DUR | 2001 | Maximum Dropout Duration. Reports the maximum dropout duration in milliseconds. |
| DROPOUT_BIN1 | 2002 | Dropout Bin 1 Counts. Counts the number of times the signal was not detected in the range of the drop_bin1 parameter. Default range is 1 to 10 milliseconds. |
| DROPOUT_BIN2 | 2003 | Dropout Bin 2 Counts. Counts the number of times the signal was not detected in the range of the drop_bin2 parameter. Default range is 11 to 20 milliseconds. |
| DROPOUT_BIN3 | 2004 | Dropout Bin 3 Counts. Counts the number of time the signal was not detected in the range of the drop_bin3 parameter. Default range is 21 to 50 milliseconds. |
| DROPOUT_BIN4 | 2005 | Dropout BIN 4 Counts. Counts the number of times the signal was not detected in the range of the drop_bin4 parameter. Default range is 51 to 100 milliseconds. |
| DROPOUT_BIN5 | 2006 | Dropout Bin 5 Counts. Counts the number of times the signal was not detected in the range of the drop_bin5 parameter. Default range is greater than 100 milliseconds. |
| No Tone Detected | 8001 | Counts the number of times the call failed due to the incoming tone not being detected by the vop_timeout. |
| Tone Lost | 8004 | Counts the number of times the call failed due to losing the incoming tone before the test_time parameter expires. |

# STANDARD REPORT CODES

| ### CALL STATISTICS | CODE | ### DEFINITION |
| --- | --- | --- |
| ORIGINATE ATTEMPT | 1 | The number of time the calling line initiated a call. |
| ORIGINATE COMPLETE | 2 | The number of times the voice path was set up the verified for the entire test time. |
| NO ALERT | 6 | The number of times that a call progresses to a point that the called line expects ringing and doesn’t get it. |
| POST DIAL DELAY | 7 | The average time (in 100ms resolution) from the last digit sent until ring (or seize) is detected on the terminate side. |
| TERMINATE ATTEMPT | 11 | The number of time the calling line initiated a call. |
| TERMINATE COMPLETE | 12 | The number of times the voice path was set up the verified for the entire test time. |
| SYSTEM ERROR | 255 (0) | System error failure due to no valid signal detection. |
| SYSTEM ERROR | 255 (1) | System error failure due to call released during signal training period. |
| SYSTEM ERROR | 255 (4) | System error failure due to result collection failure. |