CRS & AM2 SCRIPT MANUAL

For Unpaired Originate Golden Voice Basic (Analog and CAS) Script Package

(9327176E)

This document describes Unpaired Originate Golden Voice Basic Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327176E	     Unpaired Originate GV Basic (Analog and CAS)   	         O GV BASIC

This is an unpaired originate Golden Voice Basic script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired terminate Golden Voice Basic script (part # 9327177).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (T1), 128H (E1), 128H(Analog) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912182002C (Mu-Law) or 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for dial tone or seizure on the originating side and dial upon detection.    When the call is answered this call program will wait to measure a Golden Voice tone sent from the terminate call program.  When the tone is received, a set of statistics is gathered and posted which may be viewed from the Orig GV Basic report page.  Next, this call program sends a golden voice tone to the terminate call program.  If a failure occurs any time during the call, an originate attempt statistic will be incremented and a real time error will be displayed.  If the call passes the originate line will go ONHOOK.  After completion of the call the whole process starts again.

Unpaired Golden Voice Basic Originate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the calling channel. |
| digits_1 |  | The phone number that the channel will dial to ring the called line. |
| digits_2 |  | Used when more than the 15 digits of digits_1are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. |
| digits_3 |  | Used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1, 4=MFR2. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| dly_b4_meas | 6 | Amount of time, in seconds, to delay before starting the measure. Note: The expected Sendtone must start before the measure. |
| dly_b4_send | 7 | Amount of time, in seconds, to delay before sending the golden voice tone. |
| intercall | 4 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| snr_thresh | 28 | Signal To Noise Ratio Threshold. This call program will report a real time error for any SNR value below this threshold. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |
| tone_on_time | 15 | The amount of time in seconds, that the golden voice tone will be on. |

Unpaired Originate Golden Voice Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 R_ATTEMPT | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). It counts each occurrence of an OFFHOOK instruction by the originating line regardless of the success of the call. |
| 2 R_COMPLETE | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete. It counts the number of times the programmed sequence was successful in running through its complete test. If this does not increment once per ORIG ATTEMPT, you may see any of the following codes. The code received will help you identify the fault. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 7 | AVG PD DELAY | The average time from end of dialing until the first ring is detected on the called line. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits. |
| 6001 R_TOTNRGMIN | MIN. TOTAL ENERGY (-) | Reports the minimum total energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Minimum Total Energy (-)’. |
| 6002 R_TOTNRGMAX | MAX. TOTAL ENERGY (-) | Reports the maximum total energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Maximum Total Energy (-)’. |
| 6003 R_SNRMIN | MIN. SIGNAL TO NOISE RATIO | Reports the minimum Signal To Noise Ration (in dB) of the GV tone in the GV Basic Orig. Extended report page. |
| 6004 R_SNRMAX | MAX. SIGNAL TO NOISE RATIO | Reports the maximum Signal To Noise Ration (in dB) of the GV tone in the GV Basic Orig. Extended report page. |
| 6005 R_GVNRGMIN | MIN GOLDEN VOICE ENERGY (-) | Reports the minimum golden voice energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Minimum GV Energy (-)’. |
| 6006 R_GVNRGMAX | MAX. GOLDEN VOICE ENERGY (-) | Reports the maximum golden voice energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Maximum GV Energy (-)’. |
| 6007 R_SPRNRGMAX | MAX. SPURIOUS ENERGY (-) | Reports the maximum spurious energy (in dB) of the GV tone in the GV Basic Orig. Extended report page. This is a negative value, as denoted by the title ‘Maximum Spurious Energy (-)’. |
| 7025 R_SPUROVER | SPURIOUS ENERGY OVERFLOW | Reports a real-time error that the spurious energy is too high. |
| 7026 R_LOWSNR | LOW SIGNAL TO NOISE RATIO | Reports a real-time error that the Signal to Noise Ratio is too low, including the value of the low SNR, if the Signal to Noise Ratio Threshold (snr_thresh) is not met. |