CRS & AM2 SCRIPT MANUAL

For Unpaired Terminate GoldenMOS (Analog and CAS) Script Package

(9327214M)

This document describes Unpaired Terminate GoldenMOS Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327214M	     Unpaired Terminate GoldenMOS (Analog and CAS) 	        T GMOS BASIC

This is an unpaired terminate GoldenMOS script, designed for CRS & AM2 Analog and CAS units equipped with VOP3 software. This script is designed to work with the Analog Unpaired Originate GoldenMOS script (part # 9327216) and the CAS Unpaired Originate GoldenMOS script (part # 9327213).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128F or higher, LGC PCB part number 280277AY PCB  or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912181002B (Mu-Law) or 912032102B (A-Law)

- MMP LDSP Application 408 or higher.

- MMP DSP Application version 1Z or higher (280315/316 PCB).

- MMP power supply for units that use 280315/316 DSP boards (Digital units only).

- GMOS License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for ring and answer the call.  When the call is answered the Golden Voice ThreeDigitID will be sent to the originating call program to verify that the path is up.  Then a Golden Voice Tone will be sent to the originating call program to measure the round trip delay.  This call program receives a Golden Voice Tone sent back from the originate call program to complete the round trip delay measurement.  Next, this call program waits to receive the one-way delay pulse. It then uses the Golden Voice tone sent from the originating call program to measure the number of dropouts occurring in a measured time. Finally, it measures the circuit noise and Signal to noise ratio using the Golden Voice tone sent from the originating call program. When all tests pass, a set of statistics is gathered and posted which may be viewed from the Term GMOS report page.  If a failure occurs any time during the call, a terminate attempt statistic will be incremented, a real time error will be displayed and no other statistics will be collected.  The terminate line will go ONHOOK after the tone has been measured.  After completion of the call the whole process starts again.

Analog and CAS Unpaired GoldenMOS Terminate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the called channel. |
| pack_size |  | The packet size of the voice packets. Reported to the GoldenMOS application to calculate the % packet loss. Legal values are 5, 10, 15, 20, 30, 40, 50, 100ms |
| pack_size_t |  | The packet size threshold in the drop test. Must be set to the same value as the packet_size for accurate results. |
| answer_delay | 0 | Delay, in seconds, before sending ANM (Answer Message). Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 2 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to sending the initial GV tone. |
| delay_time | 3 | Amount of time, in seconds between GoldenMOS tests to allow the originating and terminating channels to re sync. |
| det_lost_sig | 0 | Flag to determine whether or not the drop test should report lost signal. Set to 1 if the report is required. |
| dly_b4_meas | 3 | Amount of time, in seconds, to delay before starting the measure. Note: The expected Sendtone must start before the measure. |
| drop_timer | 4.0 | Amount of time, in seconds to wait for the drop tone from the originatig channel. Normally this should occur within a second or so. |
| guard_time | 1 | The number of seconds that the line will ignore any incoming calls, before it checks the next incoming call. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| rpulse_timer | 4.0 | Number of seconds to wait for the GV tone sent from the other channel during a one-way delay test |
| SendIDTimeout | 5 | Number of seconds allowed for the voice path confirmation (“ThreeDigitID”) before aborting the call and incrementing the “Send ID Test Failed” counter. If the confirmation is received in the confirm time, the channel performs the remaining GoldenMOS tests and goes on-hook, then waits for another call. If the time is exceeded, the call is aborted, counter (Send ID Test Failed) is incremented, and a "1008 Send ID Test Failed" message is displayed. (If the ThreeDigitID is never confirmed, a "1009 Send ID Not Processed" message is displayed and the call is aborted.) |
| strip_timeout | 6.0 | Number of seconds to wait for the GV tone sent from the other channel during a round trip delay test |
| strip_tone | 14 | The GV tone to send back to the other channel during a round trip delay test. Should be a GV tone only. |
| test_time | 5 | The length of time in seconds the packet drop test runs. |
| TreeDigitID | 172 | This parameter determines the sequence of Golden Voice ID tones that will be sent by the termination script to verify the voice path. The id parameter specified in the origination script must match this id to complete the voice path verification. |

Unpaired Terminate GoldenMOS Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 11 R_TERMATTMPT | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 R_TERMCOMPL | TERM COMPL | The number of calls that the called line answered an incoming. This code counts the number of calls successfully answered by the specified terminate line. |
| 1008 R_SNDID_FAIL | Send ID Test Failed | The number of calls that the golden voice ID path verification check failed. |
| 1009 R_SNDID_NOTP | Send ID Not Processed | The terminating channel did not receive the confirmation that the golden voice ID tones were sent correctly. |
| 2002 R_R1WDLY_FAIL | Receive 1-Way Delay Failed | The terminating channel did not receive the one-way GV tone pulse sent from the originating channel. |
| 2003 R_SRTRIP_FAIL | Send Round Trip Delay Failed | The originating side’s round trip tone was not received at the terminating channel. |
| 2005 R_NODROPSIG | Dropout Test Signal Was Lost | Failed to receive confirmation from the terminating side that the drop test GV tone was received. |
| 2006 R_LOSTDROPSIG | Drop Test Signal Not Detected | During the drop test, the Signal Lost feature was enabled (det_lost_sig was set to 1), and a dropout greater than 200 ms was detected. |
| 2100 R_DELAY_AVER | 1-Way Avg (ms) | Reports the average receive one-way delay measured by sending the GV tone from the originating side to the terminating side of the successful calls. The value is reported in ms. |
| 2102 R_DELAY_MAX | 1-Way Max (ms) | Reports the maximum receive one-way delay measured by sending the GV tone from the originating side to the terminating side of the successful calls. The value is reported in ms. |
| 3110 TRIPDLY_AVER | RT Avg (ms) | Reports the average round trip delay for all successful calls measured by sending the GV tone from the terminating side to the originating side of the call, and back The value is reported in ms. |
| 3112 TRIPDLY_MAX | RT Max (ms) | Reports the maximum round trip delay for all successful calls measured by sending the GV tone from the terminating side to the originating side of the call, and back The value is reported in ms. |
| 3610 TERM_CHAN | Terminating Channel Indicatior | Reports whether the channel the data was received on is an originating channel, or a terminating channel. A value of 0 indicates an originating channel, a value of 1 a terminating channel. Used in the GMOS report generation to suppress the reporting of data from the originating side of the paired script. It is not possible to calculate the GoldenMOS score on the originating side of the call. |
| 5520 DROP_AVER | % Drop Avg | Reports the average of the number of dropped packets for all successful calls. Reported as an integer count. |
| 5522 DROP_MAX | % Drop Max | Reports the maximum number of dropped packets seen for a single call from all successful calls. Reported as an integer count. |
| 5530 DROP_TESTIME | Drop Test Time (ms) | This reports the duration of the drop test. The value is reported in 100ms units. With the packet size and test time, the GMOS report can calculate the 5 packet drop value required to determine the Ie value to use for the Codec. |
| 5531 DROP_PAKSIZE | Drop Packet Size (ms) | The packet size set for the dropped packet test. The reported packet size is the value set in the Call Program for the pack_size and pack_size_t variables. |
| 6001 R_TOTNRGMIN | Receive Level Min (-) | Reports the minimum total energy (in dB) of the GV tone in the GMOS CAS Paired Extended report page. This is a negative value, as denoted by the title ‘Minimum Total Energy (-)’. |
| 6002 R_TOTNRGMAX | Receive Level Max (-) | Reports the maximum total energy (in dB) of the GV tone in the GMOS CAS Paired Extended report page. This is a negative value, as denoted by the title ‘Maximum Total Energy (-)’. |
| 6003 R_SNRAVG | SNR Avg | Reports the average of all measured Signal To Noise Ratios (in dB) values of all successful calls with the GV tone in the GMOS CAS Paired Extended report page. |
| 6004 R_SNRMAX | SNR Max | Reports the maximum Signal To Noise Ratio (in dB) of the GV tone in the GMOS CAS Extended report page. |
| 6008 R_SPRNRGAVER | Circuit Noise Avg (-) | Reports the average of the measured spurious noise energy (in dB) values of the GV tone in the GMOS CAS Extended report page. This is a negative value, as denoted by the title ‘Circuit Noise Avg (-)’. |
| 6009 R_SPRNRGMIN | Circuit Noise Min (-) | Reports the minimum spurious noise energy (in dB) of the GV tone in the GMOS CAS Extended report page. This is a negative value, as denoted by the title ‘Circuit Noise Min (-)’. |