This document describes a script developed for the AM2/CRS-A and the AM2/CRS-D Bulk Call Generators. It consists of three parts.

| Part 1: Title and Description of the script | --- Page 1 |
| --- | --- |
| Part 2: Script Parameters | --- Page 1 – 2 |
| Part 3: Standard Codes | --- Page 2 |

| Script Number | Title Description | ## AM2/CRS-De Selection |
| --- | --- | --- |
| # 9327132A | Analog/CAS Unpaired Terminate VOP II Dropout Call Script | # T VOP2 DROP |

This is a Terminating VOP script that was developed to send a modulated tone for dropouts analysis.  The script will answer an incoming call and send a modulated tone for the test_time parameter or until the originating side goes onhook.  It is to be used with the O VOP2 DROP script part number 9327131.

REQUIREMENT (S):

- Host version 4.77A or higher

- FeatureCall Version 3.6 or higher

SCRIPT PARAMETERS

| PARAMETER NAME | ### RANGE | ### DEFINITION |
| --- | --- | --- |
| channel | (Analog)1-64 or 128 (Digital) A1 to D24 or A1 to D31 | The channel number of the called line. |
| answer_delay | 0-6553 | Number of seconds to delay from when the calling line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 0-6553 | Number of seconds delay from when the called line goes off hook until voice path confirmation is initiated. |
| guard_time | 1-9 | The number of seconds that the line will ignore any incoming calls, from the time that it goes on hook of the previous call. |
| ring_cycle | 0-99 | Number of seconds from the beginning of one ring cycle until the beginning of the next ring cycle. In locations where the ringing is an interrupted ring, this time should be set to bridge all quiet periods within a single ring cycle. |
| rings | 1-9 | Number of rings to be detected by the called line before it goes off hook. |
| test_time | 6553 | The time in seconds that the line will send the modulated tone. This parameter should be set 5 seconds higher than the originate Call Program’s test_time, to allow the analysis to complete on the originating line before ending the tone. |
| vop_tone | See Tone Table in Script Writers Guide | The modulated tone that is sent to the detector on the far side of the call. The tone is set to the pseudo-voice tone (Tone 14) by default. |

# STANDARD CODES

| ### CALL STATISTICS | CODE | ### DEFINITION |
| --- | --- | --- |
| TERMINATE ATTEMPT | 11 | The number of time the calling line initiated a call. |
| TERMINATE COMPLETE | 12 | The number of times the voice path was set up the verified for the entire test time. |
| NO ALERT | 6 | The number of times that a call progresses to a point that the called line expects ringing and doesn’t get it. |