CRS & AM2 SCRIPT MANUAL

For Unpaired R2 Terminate Golden Voice Basic (E1 CAS) Script Package

(9327185C)

This document describes Unpaired Terminate Golden Voice Basic Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327185C	     Unpaired R2 Terminate GV Basic (E1 CAS)   	      R2 T GV BASIC

This is an unpaired terminate Golden Voice Basic script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired originate Golden Voice Basic script (part # 9327176).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (E1) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

Call Flow

A backward signal is sent by the incoming register in response to a Forward MFR2 signal. The backward signals convey information back to the outgoing register such as status of the network and what information to send next.  After the completion of this exchange the call program will answer the call.  When the call is answered a Golden Voice Tone will be sent to the originating call program.  Next this call program will measure a Golden Voice Tone sent from the originate call program.  When the call passes, a set of statistics is gathered and posted which may be viewed from the Term GV Basic report page.  If a failure occurs any time during the call, a terminate attempt statistic will be incremented and a real time error will be displayed.   The terminate line will go ONHOOK after the tone has been measured.  After completion of the call the whole process starts again.

Unpaired Golden Voice Basic R2 Terminate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| Channel |  | The channel number of the called channel. |
| mfcbwd_dig_1 |  | MFR2 backwards digits to be sent in response to received Forward digits to complete the inter-register signaling requirements of the originating call. |
| mfcbwd_dig_2 |  | Used when more than the 15 digits of mfcbwd_dig_1 are required. |
| mfcbwd_dig_3 |  | Used when more than the 30 digits of mfcbwd_dig_2 are required. |
| answer_delay | 0 | Delay, in seconds, before sending ANM (Answer Message). Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 2 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to sending the GV tone. |
| dly_b4_meas | 8 | Amount of time, in seconds, to delay before starting the measure. Note: The expected Sendtone must start before the measure. |
| guard_time | 1 | The number of seconds that the line will ignore any incoming calls, before it checks the next incoming call. |
| level_h | -9 | Level, in dB, that the high band frequencies of MFR2 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of MF R2 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of MFR2 digits. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| snr_thresh | 28 | Signal To Noise Ratio Threshold. This call program will report a real time error for any SNR value below this threshold. |
| tone_on_time | 15 | The amount of time in seconds, that the golden voice tone will be on. |

Unpaired R2 Terminate Golden Voice Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 11 R_TERMATTMPT | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 R_TERMCOMPL | TERM COMPL | The number of calls that the called line answered an incoming call and verified the receipt of a fax. This code counts the number of calls successfully answered by the specified terminate line. |
| 6001 R_TOTNRGMIN | MIN. TOTAL ENERGY (-) | Reports the minimum total energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Minimum Total Energy (-)’. |
| 6002 R_TOTNRGMAX | MAX. TOTAL ENERGY (-) | Reports the maximum total energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Maximum Total Energy (-)’. |
| 6003 R_SNRMIN | MIN. SIGNAL TO NOISE RATIO | Reports the minimum Signal To Noise Ration (in dB) of the GV tone in the GV Basic Term. Extended report page. |
| 6004 R_SNRMAX | MAX. SIGNAL TO NOISE RATIO | Reports the maximum Signal To Noise Ration (in dB) of the GV tone in the GV Basic Term. Extended report page. |
| 6005 R_GVNRGMIN | MIN GOLDEN VOICE ENERGY (-) | Reports the minimum golden voice energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Minimum GV Energy (-)’. |
| 6006 R_GVNRGMAX | MAX. GOLDEN VOICE ENERGY (-) | Reports the maximum golden voice energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Maximum GV Energy (-)’. |
| 6007 R_SPRNRGMAX | MAX. SPURIOUS ENERGY (-) | Reports the maximum spurious energy (in dB) of the GV tone in the GV Basic Term. Extended report page. This is a negative value, as denoted by the title ‘Maximum Spurious Energy (-)’. |
| 7025 R_SPUROVER | SPURIOUS ENERGY OVERFLOW | Reports a real-time error that the spurious energy is too high. |
| 7026 R_LOWSNR | LOW SIGNAL TO NOISE RATIO | Reports a real-time error that the Signal to Noise Ratio is too low, including the value of the low SNR, if the Signal to Noise Ratio Threshold (snr_thresh) is not met. |