AM2 Niagara, AM2 Squirt and CRS SCRIPT MANUAL

For A>B Std. Script Package

(9327001B)

This document describes A>B Std. Script for AM2 Niagara, AM2 Squirt and CRS units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description		AM2 & CRS Selection

9327001B		                         A>B Std.	 	            A>B Std.

This is a paired script designed for AM2 Niagara, AM2 Squirt and CRS units which will call from one line to another (A>B).

This Script will work with all protocols for the following systems:

ANALOG

T1 CAS

E1 CAS

Call Flow

The call program will wait for dial tone or seizure on the originating side and dial upon detection.   The terminating side will wait for ringing and goes OFFHOOK when the set number of rings is reached.  When the call is answered, a SENDID/RECVID tone sequence will be used for voice path verification.  Both lines will go ONHOOK right after conversation time expires.  Once completed, the process repeats.

A Calls B Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| orig_channel |  | The number of the calling channel. |
| digits_1 |  | The phone number that the a_channel will dial to ring the b_channel. |
| digits_2 |  | Used when more than the 15 digits of b_fon_no_1are required. These digits are dialed immediately after the digits programmed in the “b_fon_no_1” field. |
| digits_3 |  | Used when more than the 30 digits of b_fon_no_1and b_fon_no_2 are required. These digits are dialed immediately after the digits programmed in the “b_fon_no_2” field. |
| answer_delay | 0.0 | Delay, in seconds, before terminating answers. Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| answer_y-1 | 0 | Whether or not reverse-battery answer supervision is required in the voice path confirmation check sequence (0=not required, 1=required). |
| term_channel |  | The number of the terminating channel. |
| call_call<s> | 0 | Time, in seconds, from the start (off hook) of one call until the start of the next call. This parameter cannot be used to gain a higher volume of calls. It is only used to “throttle back” the calling rate to a rather predictable amount (i.e. If the parameters controlling the line (and the speed of the switch under test) allowed for one completed call every six seconds, or 10 calls per minute, but, the desired rate was only four calls per minute, this parameter would be set to 15. In this manner, there would be a 15 second period between off hooks, or four calls per minute). Keep in mind that for failed calls, depending on how they fail, other timers may not allow for the line to be released (put back on hook) in time to start the next call. In this case, when the line does go on hook, it will wait the programmed intercall time and go back off hook. |
| confirm_time | 5 | Number of seconds allowed for the voice path confirmation (“id” and, if selected, “answer_y-1”) from the beginning of “cut_thru”, before aborting the call and incrementing the “no confirm” counter. If the confirmation is received in the confirm time, the channel goes on-hook, delays for the time specified by the intercall parameter, then makes another call. If the time is exceeded, the call is aborted, counter (Code 8) is incremented, and a "008 Confirming Failure" message (including time for answer signal) is logged. (If the id is never confirmed, a "255 System Error 3" message is logged and the call is aborted.) |
| cut_thru | 2.0 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay from called line going off hook until voice path confirmation check is initiated. |
| dial_delay | 0 | Number of seconds delay from the time dial tone or seizure is detected until start of dialing. |
| dial_type |  | Dial type. Selects the type of dialing. Types available, 1=Pulse 2=DTMF, 3=MFR1. |
| id | 123 | This parameter determines the sequence of ID tones that will be sent by the termination script to verify the voice path. The id parameter specified in the origination script must match this id to complete the voice path verification. Refer to the description of SENDID/RECVID. |
| intercall | 3 | Minimum time, in seconds, between on hook until next off hook. This timer works in conjunction with the call_call timer. The calling line will always be on hook a minimum of the intercall time period. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. In locations where the ringing is an interrupted ring, this time should be set to bridge all quiet periods within a single ring cycle. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| ring_timeout | 10 | Number of seconds to wait from the time that the last digit is dialed until the first ring is detected at the called line before aborting the call and incrementing the “no alert” counter. This parameter should be set at least as long as it takes the switch to process the dialed digits and ring the called line under load conditions to prevent premature aborting of a call. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| talk_time | 0 | Amount of time, in seconds, to hold up a line that is in a conversation mode (called line has answered and the voice path has been confirmed) before terminating the call (both lines going on hook). |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to go off hook until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t go off hook at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, only the initial off hook, from issuance of a “start” command, will follow the timed start delay. |

A Calls B Script Call Statistic Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 1 | ORIG ATTEMPT | The number of calls that the calling line goes off hook to initiate a call and was allowed to go back on hook without manual intervention (unit placed in stop mode). |
| 2 | ORIG COMPL | Will increment its count, up to 50000, each time the call between the two lines is complete, including voice path verification and answer supervision check, if enabled. |
| 3 | AVG START DELAY | The average time from off hook to detection of dial tone or seizure on the calling line. |
| 4 | SLOW START | The number of calls that an originating call doesn’t receive dial tone or seizure within the st_sig_dly time. This will only count if dial tone or seizure is eventually received within the st_sig_fail time. |
| 5 | NO START | The number of times that an originating call doesn’t receive dial tone or seizure within the st_sig_fail time. |
| 6 | NO ALERT | The number of times that a call progresses to the point that the called line expects ringing and doesn’t get it. |
| 7 | AVG PD DELAY | The average time from end of dialing until the first ring is detected on the called line. |
| 8 | NO CONFRM | The number of calls that the voice path verification check failed. |
| 9 | CALLED BUSY | The number of calls that busy signal was detected by the calling line after dialing all digits and waiting the "ring_timeout" period. |
| 10 | NO ANS SIGNAL | The number of calls that reverse battery was not detected at the end of the voice path confirmation check with detection enabled. |
| 11 | TERM ATTMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. |
| 12 | TERM COMPL | The number of calls that the called line answered an incoming call and verified the voice path. |