CRS & AM2 SCRIPT MANUAL

For Unpaired Terminate Golden Voice Diagnostic (Analog & CAS) Script Package

(9327189C)

This document describes Unpaired Terminate Golden Voice Diagnostic Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327189C	      Unpaired Terminate GV Diagnostic (Analog & CAS)         T GV DIAG

This is an unpaired terminate Golden Voice Diagnostic script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired originate Golden Voice Diagnostic script (part # 9327188).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (T1), 128H (E1), 128H(Analog) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912182002C (Mu-Law) or 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

ANALOG

 Loop Start, -30dBm (Ameritec part number: 9221015, revision E or newer.)

T1 CAS

 T1 E&M Wink Start, -30dBm (Ameritec part number: 9222013, revision E or newer.)

 T1 E&M Immediate Start, -30dBm (Ameritec part number: 9222014, revision E or newer.)

 T1 E&M Dial Tone Start, -30dBm (Ameritec part number: 9222015, revision E or newer.)

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

 CCITT Q411, –30dBm (Ameritec part number: 9220002, revision E or newer)

Call Flow

This call program will wait for ring and answer the call.  When the call is answered a Golden Voice Tone will be sent to the originating call program.  The originating call program measures the tone.  If spurious energy, greater than –30dB exists on the line, the originate call program will report the spurious energy’s centered frequency in a statistical bin.  If a failure occurs any time during the call, a terminate attempt statistic will be incremented and a real time error will be displayed.   The terminate line will go ONHOOK after the tone has been sent.  After completion of the call the whole process starts again.

Unpaired Golden Voice Diagnostic Terminate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the called channel. |
| answer_delay | 0 | Delay, in seconds, before sending ANM (Answer Message). Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 2 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to sending the GV tone. |
| guard_time | 1 | The number of seconds that the line will ignore any incoming calls, before it checks the next incoming call. |
| gv_tone | 0 | Selects the Golden Voice tone that will be received, match this with the originate. 0 = GV tone 29, which has frequencies centered at 1120 Hz & 1600 Hz. 1 = GV tone 30, which has frequencies centered at 2080 Hz & 2560 Hz. |
| ring_cycle | 8 | Number of seconds for the beginning of one ring cycle until the beginning of the next ring cycle. |
| rings | 1 | Number of rings to be detected by the called line before it goes off hook. |
| tone_on_time | 15 | The amount of time in seconds, that the golden voice tone will be on. |

Unpaired Terminate Golden Voice Diagnositic Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 11 R_TERMATTMPT | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 R_TERMCOMPL | TERM COMPL | The number of calls that the called line answered an incoming call and verified the receipt of a fax. This code counts the number of calls successfully answered by the specified terminate line. |