AM2/CRS-A/D(e) SCRIPT MANUAL

for Analog/CAS VoP Detection Origination Script

( 9327154C )

This document describes a script developed for the AM2/CRS-A/D(e) Bulk Call Generator. It consists of seven parts.

| Part 1: A title and description of the script | ---Page 1 |
| --- | --- |
| Part 2: Call Flow | ---Page 2 |
| Part 3: Programming Notes | ---Page 2 |
| Part 4: Description of Script Parameters | ---Page 2,3,4 |
| Part 5: Explanation of Report Codes | ---Page 5 |
| Part 6: VoP Analysis Test Specific Report Codes | ---Page 6 |
| Part 7: Report codes used by the One-way Delay test script | ---Page 7 |

| Script Number | Title Description | AM2-A/D(e) Selection |
| --- | --- | --- |
| #### 9327154C | Analog/CAS VoP Detection Originate | VoP DT C A O |

WARNING:  This script is developed to be used on Analog, T1/E1 CAS units equipped with 25-0419 VoP Option.   This script must be used with 9327155B VoP DT C A T.

This script uses DTMF digits to transmit information about the type of the test to be performed. Some Codecs may not pass the DTMF digits through the network or may corrupt them causing call failures.This Script can only be used on networks that will pass these DTMF digits through such as G.711.

REQUIREMENTS:

- Host Software Version 4.83a or higher.

- 25-0419-A/D(e) Option.

CALL FLOW

# The test_mode parameter sets the type of VoP testing you want to use on the originate side, and the terminating side automatically detects the type of testing the origination side initiates.  After the initial call setup and voice-path establishment the originate side will send a DTMF digit to the terminating side to establish the proper test format.  test_mode = 1 performs VoP Signal Analysis, test_mode = 2 performs two one way delay tests ( terminate to originate first and  then originate to terminate), and test_mode = 0 performs both tests, two one-way delay tests followed by VoP signal Analysis.  After the testing is complete, VoP Signal Analysis or Delay Test, both lines go onhook and statistics will be updated.

# PROGRAMMING NOTES

The type of testing can be selected using test_mode = 1 for VoP Signal Analysis, test_mode = 2 for two one way delay tests or test_mode = 0 for two one way delay tests followed by VoP Signal Analysis.   If any other value besides 0, 1 or 2 is used, two one way delay tests followed by VoP Signal Analysis will be performed.

By setting test_mode = 2 and duty_cycle = 100 the user can perform a dropout only test.

# SCRIPT PARAMETERS

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The analog line number of the terminating line. 1-160 A time slot in one of the four T1/E1 trunks in which voice will be sent once the call is connected. The acceptable range is A01 to D24 for a T1 unit and A01 to D31 on an E1 unit. A01 - D24, A01 - D31 |
| dial_type |  | Dial type. Types available: 1=Pulse , 2=DTMF, 3=MFR1, 4=MFR2. Normally keep this value at 2 for Analog/CAS Signaling. |
| digits_1 digits_2 digits_3 |  | The phone number that the channel will dial to ring the called line. digits_2 is used when more than the 15 digits of digits_1 are required. These digits are dialed immediately after the digits programmed in the “digits_1” field. digits_3 is used when more than the 30 digits of digits_1 and digits_2 are required. These digits are dialed immediately after the digits programmed in the “digits_2” field. |
| test_mode |  | Selects the VoP test mode. Select 1 for VoP Signal Analysis, 2 for two way one-way delay test, and 0 for two one-way delay tests followed by VoP signal Analysis. |
| answer_timer | 5.0 | Timeout value for answer supervision. |
| cut_thru | 2.0 | Time in seconds from the time the originate side sends the test selection digit and starts performing VoP Signal Analysis. If test_mode = 0 is selected, it is the time in seconds between the VoP Signal Analysis and two one-way delay tests. |
| det_lost_sig | 1 | The det_lost_sig parameter selects whether a loss of test signal will result in a failed call 200ms after the signal is lost. If the call is to be failed upon the loss of the test signal (e.g., loss of voice path), det_lost_sig should be set to 1. If the call should not be dropped, set det_lost_sig to 0. In this case, if the signal returns, dropout detection begins again when the signal is detected. |
| dial_delay | 0 | Number of seconds delay from the time dial tone is detected until start of dialing. |
| duty_cycle | 50 | The duty_cycle parameter selects the percentage of the VoP Signal Analysis Test period that is to be transmitting the signal. The parameter should be entered as a percentage. The following values are allowed: 25, 50, 75 and 100%. If 100% is selected, only dropouts are measured during the analysis period (no clipping or jitter). For unpaired calls, the duty_cycle must be the same on both the originating and terminating side of the call. |
| inter_pulse | 4 | The inter_pulse parameter selects the delay between the receive 1-way and the send 1-way delay tests on the unpaired originate script only. The inter_pulse parameter, along with the receive pulse timeout (rpulse_timer), on the terminate script can be modified together to increase the call holding time. The inter_pulse parameter should not be set lower than 4 seconds, since this will result in periodic test failures due to timing mismatches. |
| intercall | 4 | Time, in seconds, between the end of one call and the beginning of the next call. |
| level_h | -9 | Level, in dB, that the high band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_hi | 0 | Same as offset_lo, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_lo | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| off_time | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| on_time | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| rpulse_timer | 4 | The rpulse_timer parameter selects the timeout for the receive 1-way tone pulse. |
| spulse_tone | 14 | The spulse_tone parameter selects the tone that is sent during the pulse test. The tone is set to the pseudo-voice tone (Tone 14) by default. |
| st_sig_dly | 3 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the "slow start" (Code 4) counter is incremented and a "004 Slow Start Signal" error message (including time) is logged. Used to track number of slow dial tones. |
| st_sig_fail | 15 | Number of seconds to wait from off hook on the calling channel until the start signal (dial tone, wink, etc.) is received. If the time is exceeded, the call is aborted and the "no start" (Code 5) counter is incremented and a "005 No Start Signal" error message is generated. |
| test_delay | 0 | The amount of time in seconds after the line goes offhook and the script sends a digit to determine the type of testing for the terminate side. |
| test_time | 30 | The test_time parameter selects the duration of time that dropouts, clipping, and jitter are to be measured. For unpaired calls, test_time needs to be set 5 seconds lower than the terminating Call Program's test_time, to allow the analysis to complete before ending the tone. In the unpaired case, test_time selects the analysis duration on the originating side and the tone duration on the terminating side. |
| timed_start | 0 | Time, in seconds, from when the calling line is ready to initate a call until it actually does. This parameter is typically used to stagger the calls so that all originating lines don’t initiate at the same time. If the unit is set to generate calls in Synchronous Mode, the timed start will be followed for each sequence of calls. If the unit is set to generate calls in Random Mode, the timed_start will only delay before the initial off hook, from issuance of a “start” command. |
| tone_period | 1 | The tone_period parameter selects the duration of the VoP Signal Analysis Test period. The parameter should be entered in seconds. The following values are allowed: 1,2,3 or 4. For unpaired calls, the tone_period must be the same on both the originating and terminating side of the call. |
| vop_timeout | 12 | The vop_timeout parameter indicates the time that the originating side will wait for an incoming modulated tone before failing the call. |
| beclip_bin1 beclip_bin2 beclip_bin3 beclip_bin4 |  | The beclip_bin parameters are used to define the statistical bins that tally the trailing-edge clipping test results. Each test counts the number of trailing-edge clips detected during the signal analysis test stage. The number of trailing-edge clips is calculated into the trailing-edge clip average (CODE 2020). In addition, one of the statistical bins is incremented for each trailing-edge clip detected. A count is scored in the bin in which the trailing-edge clip duration falls. Each bin range is defined as a range in milliseconds. The bin trailing-edge clip ranges are as follows: Bin Clip Duration in ms (0-315ms) Default BCLIP_BIN1 0 to beclip_bin1 0 - 10 BCLIP_BIN2 (beclip_bin1 + 5ms) to bclip_bin2 11 - 20 BCLIP_BIN3 (beclip_bin2 + 5ms) to bclip_bin3 21 - 50 BCLIP_BIN4 (beclip_bin3 + 5ms) to bclip_bin4 51 – 100 BCLIP_BIN5 (beclip_bin4 + 5ms) and up 101 + |
| bin_1_max bin_2_max bin_3_max bin_4_max |  | The bin_max parameters are used to define the statistical bins which tally the dropout test results 1-Way Delay Tests. Each test counts the number of dropouts/lost packets detected during the dropout test stage. The number of dropout counts are calculated into the dropout average drops per call (CODE 2120). In addition, one of the statistical bins is incremented for each completed test. A count is scored in the bin in which the dropout count for the call falls. The bin dropout ranges are defined as follows: DROP_BIN_1 0 to bin1_max 0 DROP_BIN_2 (bin1_max + 5ms) to bin_2_max 01 - 50 DROP_BIN_3 (bin2_max + 5ms) to bin_3_max 51 - 100 DROP_BIN_4 (bin3_max + 5ms) to bin_4_max 101 - 250 DROP_BIN_5 (bin4_max + 5ms) and up 251 + |
| drop_bin1 drop_bin2 drop_bin3 drop_bin4 |  | The drop_bin parameters are used to define the statistical bins that tally the dropout test results. Each test counts the number of dropouts/lost packets detected during the dropout test stage. The number of dropouts is calculated into the dropout average (CODE 2000). In addition, one of the statistical bins is incremented for each dropout detected. A count is scored in the bin in which the dropout duration falls. Each bin range is defined as a range in milliseconds. The bin dropout ranges are defined as follows: Bin Dropout Duration in Milliseconds Default DROPOUT_BIN1 0 to drop_bin1 0 - 10 DROPOUT_BIN2 (drop_bin1 + 5ms) to drop_bin2 11 - 20 DROPOUT_BIN3 (drop_bin2 + 5ms) to drop_bin3 21 - 50 DROPOUT_BIN4 (drop_bin3 + 5ms) to drop_bin4 51 – 100 DROPOUT_BIN5 (drop_bin4 + 5ms) and up 101 + |
| feclip_bin1 feclip_bin2 feclip_bin3 feclip_bin4 |  | The feclip_bin parameters are used to define the statistical bins that tally the leading-edge clipping test results. Each test counts the number of leading-edge clips detected during the signal analysis test stage. The number of leading-edge clips is calculated into the leading-edge clip average (CODE 2010). In addition, one of the statistical bins is incremented for each leading-edge clip detected. A count is scored in the bin in which the leading-edge clip duration falls. Each bin range is defined as a range in milliseconds. The bin leading-edge clip ranges are as follows: Bin Clip Duration in ms (0-315ms) Default FCLIP_BIN1 0 to feclip_bin1 0 - 10 FCLIP_BIN2 (feclip_bin1 + 5ms) to fclip_bin2 11 - 20 FCLIP_BIN3 (feclip_bin2 + 5ms) to fclip_bin3 21 - 50 FCLIP_BIN4 (feclip_bin3 + 5ms) to fclip_bin4 51 - 100 FCLIP_BIN5 (feclip_bin4 + 5ms) and up 101 + |

REPORT CODES

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| R_ATTEMPT (CODE 1) | Origination Attempts | Counts each occurrence of an OFFHOOK occurrence regardless of the success of the call. |
| R_COMPLETE (CODE 2) | Origination Completes | Counts the number of times the programmed sequence was successful in running through its complete test. |
| R_STARTDELAY (CODE 3) | Average Start Delay | Average time it takes to receive dial tone. |
| R_SLOWSTART (CODE 4) | Slow Start Signal x | The expected signal to start dialing was not detected by the protocol within the delay timeout set by the 'st_sig_dly' parameter. |
| R_NOSTART (CODE 5) | No Start Signal x | The expected signal to start dialing was not detected by the protocol within the fail timeout set by the 'st_sig_fail' parameter. |
| R_PDTIME (CODE 7) | Avg PD Delay | Average Post Dial Delay. |
| R_NOCONFIRM (CODE 8) | No Confirm x | Counts the number of times voice path verification failed. The value of x represents the time (in 10th of seconds) voice path verfication was attempted. |

Note: See your instruction manual or the Script Writing Guide for an explanation of other report codes which may occur.

VoP ANALYSIS TEST SPECIFIC REPORT CODES

2000 AVG_DROP_DUR:  Average Dropout Duration. A running average of the dropout duration in milliseconds.

2001 MAX_DROP_DUR: Maximum Dropout Duration. Reports the maximum dropout  duration in milliseconds.

2002 DROPOUT_BIN1:  Dropout Bin 1 Counts. Counts the number of dropouts detected in the range of the drop_bin1 parameter (0 to 10ms).

2003 DROPOUT_BIN2:  Dropout Bin 2 Counts. Counts the number of dropouts detected in the range from the drop_bin1 parameter to the drop_bin2 parameter (11 to 20ms).

2004 DROPOUT_BIN3:  Dropout Bin 3 Counts. Counts the number of dropouts detected in the range from the drop_bin2 parameter to the drop_bin3 parameter (21 to 50ms).

2005 DROPOUT_BIN4: Dropout Bin 4 Counts. Counts the number of dropouts detected in the range from the drop_bin3 parameter to the drop_bin4 parameter (51 to 100ms).

2006 DROPOUT_BIN5: Dropout Bin 5 Counts. Counts the number of dropouts detected in the range from the drop_bin4 parameter to the drop_bin5 parameter (greater than 100ms).

2010 AVG_FCLIP_DUR: Average Leading-Edge Clip Duration. A running average of the leading-edge clipping duration in milliseconds.

2011 MAX_FCLIP_DUR:Maximum Leading-Edge Clip Duration. Reports the maximum leading-edge clipping duration in milliseconds.

2012 FCLIP_BIN1: Leading-Edge Clip Bin 1 Counts. Counts the number of leading edge clips detected in the range of the feclip_bin1 parameter (0to 10ms).

2013 FCLIP_BIN2: Leading-Edge Clip Bin 2 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin1 parameter to the feclip_bin2 parameter (11 to 20ms).

2014 FCLIP_BIN3: Leading-Edge Clip Bin 3 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin2 parameter to the feclip_bin3 parameter (21 to 50ms).

2015 FCLIP_BIN4: Leading-Edge Clip Bin 4 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin3 parameter to the feclip_bin4 parameter (51 to 100ms).

2016 FCLIP_BIN5: Leading-Edge Clip Bin 5 Counts. Counts the number of leading edge clips detected in the range from the feclip_bin4 parameter to the feclip_bin5 parameter (greater than 100ms).

2020 AVG_BCLIP_DUR: Average Trailing-Edge Clip Duration. A running average of the trailing-edge clipping duration in milliseconds.

2021 MAX_BCLIP_DUR: Maximum Trailing-Edge Clip Duration. Reports the maximum trailing-edge clipping duration in milliseconds.

2022 BCLIP_BIN1: Trailing-Edge Clip Bin 1 Counts. Counts the number of trailing edge clips detected in the range of the beclip_bin1 parameter (0 to 10ms).

2023 BCLIP_BIN2: Trailing-Edge Clip Bin 2 Counts. Counts the number of trailing edge clips detected in the range from the beclip_bin1 parameter to the beclip_bin2 parameter (11 to 20ms).

2024 BCLIP_BIN3: Trailing-Edge Clip Bin 3 Counts. Counts the number of trailing  edge clips detected in the range from the beclip_bin2  parameter to the beclip_bin3 parameter (21 to 50ms).

2025 BCLIP_BIN4: Trailing-Edge Clip Bin 4 Counts. Counts the number of trailing edge clips detected in the range from the beclip_bin3 parameter to the beclip_bin4 parameter (51 to 100ms).

2026 BCLIP_BIN5: Trailing-Edge Clip Bin 5 Counts. Counts the number of trailing edge clips detected in the range from the beclip_bin4 parameter to the beclip_bin5 parameter (greater than 100ms).

2030 AVER_JITTER: Average Jitter Duration. Records the average jitter duration in milliseconds.

2031 MAX_JITTER: Maximum Jitter Duration. Records the maximum jitter duration in milliseconds.

All measurements are per channel or line.

REPORT CODES USED BY THE ONE-WAY DELAY TEST

2100 AVER_DELAY: Average 1-Way Delay. Reports the average one-way delay in milliseconds.

2101 MIN_DELAY: Minimum 1-Way Delay. Reports the minimum one-way delay in milliseconds.

2102 MAX_DELAY: Maximum 1-Way Delay. Reports the maximum one-way delay in milliseconds.

2103 DELAY_BIN_1: 1-Way Delay Bin 1 Counts. 100 - counts the number of calls where the one-way delay is less than 100 milliseconds.

2104 DELAY_BIN_2: 1-Way Delay Bin 2 Counts. 200 - counts the number of calls where the one-way delay is between 101 and 200 milliseconds.

2105 DELAY_BIN_3: 1-Way Delay Bin 3 Counts. 300 - counts the number of calls where the one-way delay is between 201 and 300 milliseconds.

2106 DELAY_BIN_4: 1-Way Delay Bin 4 Counts. 400 - counts the number of calls where the one-way delay is between 301 and 400 milliseconds.

2107 DELAY_BIN_5: 1-Way Delay Bin 5 Counts. Counts the number of calls where the one-way delay is greater than 401 milliseconds.