CRS & AM2 SCRIPT MANUAL

For Unpaired R2 Terminate Golden Voice ID (E1 CAS) Script Package

(9327187C)

This document describes Unpaired Terminate Golden Voice ID Script for CRS & AM2 units.  It consists of four parts.

Part 1: A title and description of the script

Part 2: Call Flow

Part 3: Description of Script Parameters

Part 4: Explanation of Report Codes

Script Number	      	      Title Description			AM2 & CRS Selection

9327187C	     	 Unpaired R2 Terminate GV ID (E1 CAS)   	      R2 T GV ID

This is an unpaired terminate Golden Voice ID script, designed for CRS & AM2 units equipped with VOP3 software. This script is designed to work with the unpaired originate Golden Voice ID script (part # 9327194).

REQUIREMENTS: Unit must be able to support extended reporting.

- Host software version 5.07A or higher.

- LGC software version 128H (E1) or higher, LGC PCB part number 280277AY PCB or 280093 revision H or higher.

- DSP software version 101M or higher, DSP hardware D7 or higher (280105AY), or NC6 (280257AY)

- Tone Generator A software 912032102C (A-Law)

- MMP LDSP Application 4.10 or higher.

    MMP DSP Application version 1Z or higher (280315/316 PCB).

    MMP power supply for units that use 280315/316 DSP boards (Digital units only).

   VOP3 License Keys for the unit.

This Script will work with protocols for the following systems:

E1 CAS

 R2 CCITT Q421/Q440 (Ameritec part number: 9220002, revision N or newer)

Call Flow

A backward signal is sent by the incoming register in response to a Forward MFR2 signal. The backward signals convey information back to the outgoing register such as status of the network and what information to send next.  After the completion of this exchange the call program will answer the call.  When the call is answered, the Golden Voice SENDID/RECVID tone sequence will be used for voice path verification. Conversation time is controlled by the originating side. Failure to verify voice path will end the test sequence.  The terminate line will go ONHOOK after receiving the Golden Voice ID tone from the originate.  After completion of the call the whole process starts again.

Unpaired Golden Voice ID R2 Terminate Script Parameters

| PARAMETER | DEFAULT | DESCRIPTION |
| --- | --- | --- |
| channel |  | The channel number of the called channel. |
| mfcbwd_dig_1 |  | MFR2 backwards digits to be sent in response to received Forward digits to complete the inter-register signaling requirements of the originating call. |
| mfcbwd_dig_2 |  | Used when more than the 15 digits of mfcbwd_dig_1 are required. |
| mfcbwd_dig_3 |  | Used when more than the 30 digits of mfcbwd_dig_2 are required. |
| answer_delay | 0 | Delay, in seconds, before sending ANM (Answer Message). Number of seconds to delay from when the called line has detected and met the desired number of rings until it actually goes off hook. |
| confirm_time | 10 | Maximum time to wait for the initial voice path (id) verifcation test to complete (see report code 8). |
| cut_thru | 2 | Amount of time, in seconds, to wait for the voice channel to be cut through. Number of seconds delay prior to sending the GV tone. |
| guard_time | 1 | The number of seconds that the line will ignore any incoming calls, before it checks the next incoming call. |
| id | 172 | The 3-Golden Voice tone sequence used for confirmation of voice path. The range for each digit is 0-9. The corresponding frequency for each digit: 0=1025Hz, 1=1150Hz, 2=1275Hz, 3=1400Hz, 4=1525Hz, 5=1650Hz, 6=1775Hz, 7=1900Hz, 8=2025Hz, and 9=2150Hz. All tones levels are at –12dBm. |
| level_h | -9 | Level, in dB, that the high band frequencies of MFR2 digits are generated. Typical telephones are fixed to send -9dB. |
| level_l | -9 | Level, in dB, that the low band frequencies of MF R2 digits are generated. Typical telephones are fixed to send -9dB. |
| offset_h | 0 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| offset_l | 0 | Percent of frequency offset, in tenths of a percent, applied to the low band of MFR2 digits. |
| offtime | 60 | The amount of time, in milliseconds, of quiet time following the “ontime” of each tone dialed digit. |
| onhk_dly | 1.5 | The amount of time, in seconds, that the call program delays before going onhook. |
| ontime | 60 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |

Unpaired R2 Terminate Golden Voice ID Script Data Register Codes

| CODE | TITLE | DESCRIPTION |
| --- | --- | --- |
| 11 | TERM ATTEMPT | The number of calls that the called line goes off hook as a result of receiving the appropriate number of rings. This code will count the number of times the specified line has detected an incoming call and the programmed number of expected rings, then go off hook regardless of the success of the call. |
| 12 | TERM COMPL | The number of calls that the called line answered an incoming call and verified the receipt of a fax. This code counts the number of calls successfully answered by the specified terminate line. |
| 8 | No Confirm | The voice path verification test failed. If the error also occurs on the originating side, voice path was never established. |
| 255 | System Error 2 | The call program failed to initiate a SENDID command. |
| 255 | System Error 3 | The call program failed to initiate a RECVID command. |