This document describes a script developed for the AM2/CRS-De Bulk Call Generators. It consists of three parts.

| Part 1: Title and Description of the script | --- Page 1 |
| --- | --- |
| Part 2: Script Parameters | --- Page 1 – 2 |
| Part 3: Standard Codes | --- Page 2 |

| Script Number | Title Description | AM2/CRS-De Selection |
| --- | --- | --- |
| # 9327133A | Unpaired R2 Terminate VOP II Dropout Call Script | # R2T VOP2 DRP |

This is a Terminating VOP script that was developed to send a modulated tone for dropout analysis.  The script will automatically respond to the incoming R2 (MFR2) digits by dialing out a preprogrammed string of MFR2 digits. It will then answer and  send a modulated tone for the test_time parameter or until the originating side goes onhook.  It is to be used with the O VOP2 DROP script part number 9327131.

REQUIREMENT (S):

- Host version 4.77A or higher

- FeatureCall Version 3.6 or higher

SCRIPT PARAMETERS

| PARAMETER NAME | ### RANGE | ### DEFINITION |
| --- | --- | --- |
| channel | A1 - D31 | The AM2-De or CRS-De channel number of the terminating line. |
| mfcbwd_1 | 0-9, A ,B ,C, D ,E | The phone number that the channel will dial to ring the called line. |
| mfcbwd_2 | Same as mfcbwd_1 | Used when more than the 15 digits of mfcbwd_1 are required. These digits are dialed immediately after the digits programmed into the “mfcbwd_1” field. |
| mfcbwd_3 | Same as mfcbwd_1 | Used when more than 30 digits of mfcbwd_1 and mfcbwd_2 are required. These digits are dialed immediately after the digits programmed in the “mfcbwd_2” field. |
| answer_delay | 0-6553 | Number of seconds to delay from when the calling line has detected and met the desired number of rings until it actually goes off hook. |
| cut_thru | 0-6553 | Number of seconds delay from when the called line goes off hook until voice path confirmation is initiated. |
| guard_time | 1-9 | The number of seconds that the line will ignore any incoming calls, from the time that it goes on hook of the previous call. |
| level_l | 0 to –30 | Level, in dB, that the low band frequencies of DTMF or MF R1 digits are generated, Typical telephones are fixed to send –9dB. |
| level_h | 0 to –30 | Same as level_l, but for the high band. The difference between the setting in level_l and level_h, if any, is referred to as the amount of “twist”, expressed in dB. |
| offset_l | -9.9 to 9.9 | Percent of frequency offset, in tenths of a percent, applied to the low band of DTMF or MF R1 digits. |
| offset_h | -9.9 to 9.9 | Same as offset_l, but for the high band. Both the level and offset parameters are used to limit test the tone receivers of a switch. Under normal load test conditions, these parameters are left at their defaults. |
| ontime | 5-1275 | The amount of time, in milliseconds, that the dual tones of a tone dialed digit are present for each digit. |
| offtime | 5-1275 | The amount of time, in milliseconds, of quite time following the “ontime” of each tone dialed digit. |
| test_time | 6553 | The time in seconds that the line will send the modulated tone. This parameter should be set 5 seconds higher than the originate Call Program’s test_time, to allow the analysis to complete on the originating line before ending the tone. |
| vop_tone | See Tone Table in Script Writers Guide | The modulated tone that is sent to the detector on the far side of the call. The tone is set to the pseudo-voice tone (Tone 14) by default. |

# STANDARD CODES

| ### CALL STATISTICS | CODE | ### DEFINITION |
| --- | --- | --- |
| TERMINATE ATTEMPT | 11 | The number of time the calling line initiated a call. |
| TERMINATE COMPLETE | 12 | The number of times the voice path was set up the verified for the entire test time. |
| NO ALERT | 6 | The number of times that a call progresses to a point that the called line expects ringing and doesn’t get it. |